SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

| Product form | : | Mixture |
| --- | --- | --- |
| Product name | : | JAPANESE BAMBOO #EU47242F 25% in DPG |
| Product code | : | EU47242F_25% |
| Type of product | : | Perfumes, Fragrances |

1.2. Relevant identified uses of the substance or mixture and uses advised against

1.2.1. Relevant identified uses

| Industrial/Professional use spec | : | Industrial For professional use only |
| --- | --- | --- |
| Use of the substance/mixture | : | Perfumes, Fragrances |
| Function or use category | : | Odour agents |

1.2.2. Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

No additional information available

1.4. Emergency telephone number

No additional information available

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

| Skin sensitization, Category 1 | H317 |  |  |
| --- | --- | --- | --- |
| Hazardous to the aquatic environment – Chronic Hazard Category 3 | H412 |  |  |

Full text of H- and EUH-statements: see section 16

Adverse physicochemical, human health and environmental effects

No additional information available

2.2. Label elements

Labeling according to Regulation (EC) No. 1272/2008 [CLP]

| Hazard pictograms (CLP) | : |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GHS07 |  |  |  |  |  |
| Signal word (CLP) | : | Warning |  |  |  |  |  |
| Contains | : | Hexyl cinnamic aldehyde; Cyclamal; Melonal; Hydroxy; Helional; Amyl cinnamic aldehyde; Hexyl salicylate; Eugenol; isoeugenol; Citronellol Pure; Methyl heptine carbonate; Floralozone; Triplal (Vertocitral); 3-(2,2-dimethyl-3-hydroxypropyl)toluene, (alt.): 2,2-dimethyl-3-(3-methylphenyl)propanol; Linalool; Geraniol; Nerol |  |  |  |  |  |
| Hazard statements (CLP) | : | H317 - May cause an allergic skin reaction. H412 - Harmful to aquatic life with long lasting effects. |  |  |  |  |  |
| Precautionary statements (CLP) | : | P261 - Avoid breathing dust/fume/gas/mist/vapors/spray. P272 - Contaminated work clothing should not be allowed out of the workplace. P273 - Avoid release to the environment. P280 - Wear protective gloves/protective clothing/eye protection/face protection/hearing protection. P302+P352 - IF ON SKIN: Wash with plenty of water. P321 - Specific treatment (see supplemental first aid instruction on this label). |  |  |  |  |  |
| Extra phrases | : | Restricted to professional users. |  |  |  |  |  |

2.3. Other hazards

Contains no PBT and/or vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII

The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or substance(s) are not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %

SECTION 3: Composition/Information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Linalool | CAS-No.: 78-70-6 EC-No.: 201-134-4 EC Index-No.: 603-235-00-2 REACH-no: 01-2119474016-42 | 0.925 – 1.825 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| benzyl benzoate | CAS-No.: 120-51-4 EC-No.: 204-402-9 EC Index-No.: 607-085-00-9 REACH-no: 01-2119976371-33 | 0.8 – 1.6175 | Acute Tox. 4 (Oral), H302 Aquatic Acute 1, H400 Aquatic Chronic 2, H411 |
| Methyl ionone (mixture of isomers) | CAS-No.: 1335-46-2 EC-No.: 215-635-0 | 0.65 – 1.3 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Aquatic Chronic 2, H411 |
| Undecavertol | CAS-No.: 81782-77-6 EC-No.: 279-815-0 | 0.575 – 1.15 | Aquatic Acute 1, H400 Aquatic Chronic 2, H411 |
| Hexyl cinnamic aldehyde | CAS-No.: 101-86-0 EC-No.: 202-983-3 REACH-no: 01-2119533092-50 | 0.5 – 1 | Skin Sens. 1, H317 Aquatic Chronic 2, H411 |
| Citronellol Pure | CAS-No.: 106-22-9 EC-No.: 203-375-0 REACH-no: 01-2119453995-23 | 0.45 – 0.9 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| Amyl cinnamic aldehyde | CAS-No.: 122-40-7 EC-No.: 204-541-5 | 0.45 – 0.8875 | Skin Sens. 1B, H317 Aquatic Chronic 2, H411 |
| Helional | CAS-No.: 1205-17-0 EC-No.: 214-881-6 REACH-no: 01-2120740119-58 | 0.425 – 0.855 | Skin Sens. 1B, H317 Repr. 2, H361 Aquatic Chronic 2, H411 |
| 3-(2,2-dimethyl-3-hydroxypropyl)toluene, (alt.): 2,2-dimethyl-3-(3-methylphenyl)propanol | CAS-No.: 103694-68-4 EC-No.: 403-140-4 EC Index-No.: 603-138-00-5 | 0.02875 – 0.8225 | Skin Sens. 1, H317 Aquatic Chronic 3, H412 |
| Benzyl acetate substance with national workplace exposure limit(s) (BE, DK, ES, IE, LT, LV, PT, RO) | CAS-No.: 140-11-4 EC-No.: 205-399-7 REACH-no: 01-2119638272-42 | 0.35 – 0.7125 | Aquatic Chronic 3, H412 |
| Carbitol substance with national workplace exposure limit(s) (AT, DE, EE, SE, SI, CH) | CAS-No.: 111-90-0 EC-No.: 203-919-7 REACH-no: 01-2119475105-42 | 0.2712375 – 0.542475 | Not classified |
| Hydroxy | CAS-No.: 107-75-5 EC-No.: 203-518-7 REACH-no: 01-2119973482-31 | 0.279025 – 0.53305 | Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| Bacdanol | CAS-No.: 28219-61-6 EC-No.: 248-908-8 REACH-no: 01-2119529224-45 | 0.25 – 0.5 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Aquatic Chronic 1, H410 |
| Geraniol | CAS-No.: 106-24-1 EC-No.: 203-377-1 EC Index-No.: 603-241-00-5 REACH-no: 01-2119552430-49 | 0.24 – 0.4725 | Skin Irrit. 2, H315 Eye Dam. 1, H318 Skin Sens. 1, H317 |
| Hexyl salicylate | CAS-No.: 6259-76-3 EC-No.: 228-408-6 | 0.2 – 0.376925 | Skin Irrit. 2, H315 Skin Sens. 1, H317 Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| Floralozone | CAS-No.: 67634-15-5 EC-No.: 266-819-2 REACH-no: 01-2120758796-34 | 0.2 – 0.375 | Aquatic Acute 1, H400 Aquatic Chronic 2, H411 Skin Irrit. 2, H315 Skin Sens. 1B, H317 |
| Nerol | CAS-No.: 106-25-2 EC-No.: 203-378-7 | 0.16 – 0.315 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1, H317 |
| ethanol, ethyl alcohol substance with national workplace exposure limit(s) (AT, BE, BG, CZ, DE, DK, EE, ES, FI, FR, GB, GR, HR, HU, IE, LT, LV, NL, PL, PT, RO, SE, SI, SK, NO, CH) | CAS-No.: 64-17-5 EC-No.: 200-578-6 EC Index-No.: 603-002-00-5 | 0.125 – 0.2375 | Flam. Liq. 2, H225 |
| Cyclamal | CAS-No.: 103-95-7 EC-No.: 203-161-7 REACH-no: 01-2119970582-32 | 0.075375 – 0.165 | Skin Irrit. 2, H315 Skin Sens. 1B, H317 Aquatic Chronic 2, H411 |
| Triplal (Vertocitral) | CAS-No.: 68039-49-6 EC-No.: 268-264-1 | 0.075 – 0.1572 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1, H317 Aquatic Chronic 3, H412 |
| Melonal | CAS-No.: 106-72-9 EC-No.: 203-427-2 | 0.05 – 0.122675 | Skin Sens. 1B, H317 |
| Eugenol | CAS-No.: 97-53-0 EC-No.: 202-589-1 REACH-no: 01-2119971802-33 | 0.05 – 0.115 | Acute Tox. 4 (Oral), H302 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| Sandal Mysore Core | CAS-No.: 28219-60-5 EC-No.: 248-907-2 | 0.05 – 0.1 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| Cyclogalbanate (Allyl Cyclohexyl Glycolate) | CAS-No.: 68901-15-5 EC-No.: 272-657-3 REACH-no: 01-2120770514-54 | 0.05 – 0.1 | Acute Tox. 4 (Oral), H302 Aquatic Chronic 1, H410 |
| isoeugenol | CAS-No.: 97-54-1 EC-No.: 202-590-7 EC Index-No.: 604-094-00-X; 202-590-1 REACH-no: 17-2119417630-49 | 0.025 – 0.0625 | Acute Tox. 4 (Oral), H302 Acute Tox. 4 (Dermal), H312 Acute Tox. 4 (Inhalation), H332 Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1A, H317 Carc. 2, H351 STOT SE 3, H335 |
| Diphenyl oxide substance with national workplace exposure limit(s) (AT, BE, BG, CY, CZ, DE, DK, EE, ES, FI, FR, GB, GI, GR, HR, HU, IE, IT, LT, LU, LV, MT, NL, PL, PT, RO, SE, SI, SK, NO, CH); substance with a Community workplace exposure limit | CAS-No.: 101-84-8 EC-No.: 202-981-2 REACH-no: 01-2119472545-33 | 0 – 0.0125 | Eye Irrit. 2, H319 Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| Methyl heptine carbonate | CAS-No.: 111-12-6 EC-No.: 203-836-6 | 0 – 0.01 | Aquatic Acute 1, H400 Aquatic Chronic 3, H412 Skin Sens. 1A, H317 |
| Alcohol C-10 substance with national workplace exposure limit(s) (BG, DE, LT, LV, RO, CH) | CAS-No.: 112-30-1 EC-No.: 203-956-9 | 0 – 0.00175 | Aquatic Chronic 3, H412 |
| Aldehyde C-6 substance with national workplace exposure limit(s) (FI, PL) | CAS-No.: 66-25-1 EC-No.: 200-624-5 | 0 – 0.00045 | Flam. Liq. 3, H226 |
| Caproic acid substance with national workplace exposure limit(s) (BG, LT, LV) | CAS-No.: 142-62-1 EC-No.: 205-550-7 | 0 – 0.000025 | Eye Dam. 1, H318 Skin Corr. 1C, H314 |

| Specific concentration limits: |  |  |
| --- | --- | --- |
| Name | Product identifier | Specific concentration limits (%) |
| isoeugenol | CAS-No.: 97-54-1 EC-No.: 202-590-7 EC Index-No.: 604-094-00-X; 202-590-1 REACH-no: 17-2119417630-49 | (0.01 ≤ C ≤ 100) Skin Sens. 1A, H317 |

Full text of H- and EUH-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

| First-aid measures general | : | Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label where possible). |
| --- | --- | --- |
| First-aid measures after inhalation | : | Allow affected person to breathe fresh air. Allow the victim to rest. |
| First-aid measures after skin contact | : | Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse. |
| First-aid measures after eye contact | : | Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persists. |
| First-aid measures after ingestion | : | Rinse mouth. Do NOT induce vomiting. Obtain emergency medical attention. |

4.2. Most important symptoms and effects, both acute and delayed

| Symptoms/effects | : | Not expected to present a significant hazard under anticipated conditions of normal use. |
| --- | --- | --- |

4.3. Indication of any immediate medical attention and special treatment needed

No additional information available

SECTION 5: Firefighting measures

5.1. Extinguishing media

| Suitable extinguishing media | : | Foam. Dry powder. Carbon dioxide. Water spray. Sand. |
| --- | --- | --- |
| Unsuitable extinguishing media | : | Do not use a heavy water stream. |

5.2. Special hazards arising from the substance or mixture

No additional information available

5.3. Advice for firefighters

| Firefighting instructions | : | Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. Prevent fire-fighting water from entering environment. |
| --- | --- | --- |
| Protection during firefighting | : | Do not enter fire area without proper protective equipment, including respiratory protection. |

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

6.1.1. For non-emergency personnel

| Emergency procedures | : | Evacuate unnecessary personnel. |
| --- | --- | --- |

6.1.2. For emergency responders

| Protective equipment | : | Equip cleanup crew with proper protection. |
| --- | --- | --- |
| Emergency procedures | : | Ventilate area. |

6.2. Environmental precautions

Prevent entry to sewers and public waters. Notify authorities if liquid enters sewers or public waters.

6.3. Methods and material for containment and cleaning up

| Methods for cleaning up | : | Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Collect spillage. Store away from other materials. |
| --- | --- | --- |

6.4. Reference to other sections

See Heading 8. Exposure controls and personal protection.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

| Precautions for safe handling | : | Wash hands and other exposed areas with mild soap and water before eating, drinking or smoking and when leaving work. Provide good ventilation in process area to prevent formation of vapor. |
| --- | --- | --- |

7.2. Conditions for safe storage, including any incompatibilities

| Storage conditions | : | Keep only in the original container in a cool, well ventilated place away from : Keep container closed when not in use. |
| --- | --- | --- |
| Incompatible products | : | Strong bases. Strong acids. |
| Incompatible materials | : | Sources of ignition. Direct sunlight. |
| Germany |  |  |
| Storage class (LGK, TRGS 510) | : | LGK 12 - Non-combustible liquids |
| Joint storage table | : |  |
| Joint storage not permitted for | : | LGK 1, LGK 6.2, LGK 7 |
| Joint storage with restrictions permitted for | : | LGK 4.1A, LGK 4.3, LGK 5.1C |
| Joint storage permitted for | : | LGK 2A, LGK 2B, LGK 3, LGK 4.1B, LGK 4.2, LGK 5.1A, LGK 5.1B, LGK 5.2, LGK 6.1A, LGK 6.1B, LGK 6.1C, LGK 6.1D, LGK 8A, LGK 8B, LGK 10, LGK 11, LGK 12, LGK 13, LGK 10-13 |
| Switzerland |  |  |
| Storage class (LK) | : | LK 10/12 - Liquids |

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

8.1.1 National occupational exposure and biological limit values

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| EU - Indicative Occupational Exposure Limit (IOEL) |  |
| IOEL TWA | 7 mg/m³ |
| IOEL TWA | 1 ppm |
| IOEL STEL | 14 mg/m³ |
| IOEL STEL | 2 ppm |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 7 mg/m³ |
| MAK (OEL TWA) | 1 ppm |
| MAK (OEL STEL) | 14 mg/m³ |
| MAK (OEL STEL) | 2 ppm |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ (vapor) |
| OEL TWA | 1 ppm (vapor) |
| OEL STEL | 14 mg/m³ (vapor) |
| OEL STEL | 2 ppm (vapor) |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 7 mg/m³ |
| GVI (OEL TWA) | 1 ppm |
| KGVI (OEL STEL) | 14 mg/m³ |
| KGVI (OEL STEL) | 2 ppm |
| Cyprus - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 5 mg/m³ |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 7 mg/m³ |
| HTP (OEL TWA) | 1 ppm |
| HTP (OEL STEL) | 14 mg/m³ |
| HTP (OEL STEL) | 2 ppm |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 7 mg/m³ (indicative limit) |
| VME (OEL TWA) | 1 ppm (indicative limit) |
| VLE (OEL C/STEL) | 14 mg/m³ (indicative limit) |
| VLE (OEL C/STEL) | 2 ppm (indicative limit) |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 7.1 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed-vapor) |
| AGW (OEL TWA) | 1 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed-vapor) |
| Gibraltar - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 200 ppm |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 7 mg/m³ |
| CK (OEL STEL) | 14 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ (vapour) |
| OEL TWA | 1 ppm (vapour) |
| OEL STEL | 14 mg/m³ (vapour) |
| OEL STEL | 2 ppm (vapour) |
| Italy - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 7 mg/m³ |
| IPRV (OEL TWA) | 1 ppm |
| TPRV (OEL STEL) | 14 mg/m³ |
| TPRV (OEL STEL) | 2 ppm |
| Luxembourg - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Malta - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 7 mg/m³ |
| TGG-8u (OEL TWA) | 1 ppm |
| TGG-15min (OEL STEL) | 14 mg/m³ |
| TGG-15min (OEL STEL) | 2 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 7 mg/m³ |
| NDSCh (OEL STEL) | 14 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm (vapor) |
| OEL STEL | 14 mg/m³ (indicative limit value) |
| OEL STEL | 2 ppm (indicative limit value-vapor) |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) | 7 mg/m³ |
| NPHV (OEL TWA) | 1 ppm |
| NPHV (OEL C) | 7.1 mg/m³ |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 7.1 mg/m³ (vapor) |
| VLA-ED (OEL TWA) | 1 ppm (vapor) |
| VLA-EC (OEL STEL) | 14.2 mg/m³ (vapor) |
| VLA-EC (OEL STEL) | 2 ppm (vapor) |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 7 mg/m³ |
| NGV (OEL TWA) | 1 ppm |
| KGV (OEL STEL) | 14 mg/m³ |
| KGV (OEL STEL) | 2 ppm |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) | 7 mg/m³ |
| WEL TWA (OEL TWA) | 1 ppm |
| WEL STEL (OEL STEL) | 14 mg/m³ |
| WEL STEL (OEL STEL) | 2 ppm |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 7 mg/m³ |
| Grenseverdi (OEL TWA) | 1 ppm |
| Korttidsverdi (OEL STEL) | 14 mg/m³ (value from the regulation) |
| Korttidsverdi (OEL STEL) | 2 ppm (value from the regulation) |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 7 mg/m³ (aerosol, vapour) |
| MAK (OEL TWA) | 1 ppm (aerosol, vapour) |
| KZGW (OEL STEL) | 14 mg/m³ (aerosol, vapour) |
| KZGW (OEL STEL) | 2 ppm (aerosol, vapour) |
| OEL chemical category | Category 2 reproductive toxin |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 1 ppm (vapor) |
| ACGIH OEL STEL | 2 ppm (vapor fraction) |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 66 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) | 10 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 10 mg/m³ |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 100 mg/m³ |
| OEL TWA | 15 ppm |
| OEL STEL | 200 mg/m³ |
| OEL STEL | 30 ppm |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 66 mg/m³ (aerosol, vapour) |
| MAK (OEL TWA) | 10 ppm (aerosol, vapour) |
| KZGW (OEL STEL) | 66 mg/m³ (aerosol, vapour) |
| KZGW (OEL STEL) | 10 ppm (aerosol, vapour) |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 62 mg/m³ |
| OEL TWA | 10 ppm |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 61 mg/m³ |
| OEL TWA | 10 ppm |
| OEL STEL | 122 mg/m³ |
| OEL STEL | 20 ppm |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 10 ppm |
| OEL STEL | 30 ppm (calculated) |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 5 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 10 ppm |
| OEL chemical category | A4 - Not Classifiable as a Human Carcinogen |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 50 mg/m³ |
| OEL TWA | 8 ppm |
| OEL STEL | 80 mg/m³ |
| OEL STEL | 13 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 62 mg/m³ |
| VLA-ED (OEL TWA) | 10 ppm |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 10 ppm |
| ACGIH chemical category | Not Classifiable as a Human Carcinogen |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 5 mg/m³ |

| ethanol, ethyl alcohol (64-17-5) |  |
| --- | --- |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 1900 mg/m³ |
| MAK (OEL TWA) | 1000 ppm |
| MAK (OEL STEL) | 3800 mg/m³ |
| MAK (OEL STEL) | 2000 ppm |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 1907 mg/m³ |
| OEL TWA | 1000 ppm |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 1000 mg/m³ |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 1900 mg/m³ |
| GVI (OEL TWA) | 1000 ppm |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 1000 mg/m³ |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 1900 mg/m³ |
| OEL TWA | 1000 ppm |
| OEL STEL | 3800 mg/m³ |
| OEL STEL | 2000 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 1000 mg/m³ |
| OEL TWA | 500 ppm |
| OEL STEL | 1900 mg/m³ |
| OEL STEL | 1000 ppm |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 1900 mg/m³ |
| HTP (OEL TWA) | 1000 ppm |
| HTP (OEL STEL) | 2500 mg/m³ |
| HTP (OEL STEL) | 1300 ppm |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 1900 mg/m³ |
| VME (OEL TWA) | 1000 ppm |
| VLE (OEL C/STEL) | 9500 mg/m³ |
| VLE (OEL C/STEL) | 5000 ppm |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 380 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) | 200 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 1900 mg/m³ |
| OEL TWA | 1000 ppm |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 1900 mg/m³ |
| CK (OEL STEL) | 3800 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL STEL | 1000 ppm |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 1000 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 1000 mg/m³ |
| IPRV (OEL TWA) | 500 ppm |
| TPRV (OEL STEL) | 1900 mg/m³ |
| TPRV (OEL STEL) | 1000 ppm |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 260 mg/m³ |
| TGG-8u (OEL TWA) | 137 ppm |
| TGG-15min (OEL STEL) | 1900 mg/m³ |
| TGG-15min (OEL STEL) | 1000 ppm |
| MAC chemical category | skin notation |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 1900 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL STEL | 1000 ppm |
| OEL chemical category | A3 - Confirmed Animal Carcinogen with Unknown Relevance to Humans |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 1900 mg/m³ |
| OEL TWA | 1000 ppm |
| OEL STEL | 9500 mg/m³ |
| OEL STEL | 5000 ppm |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) | 960 mg/m³ |
| NPHV (OEL TWA) | 500 ppm |
| NPHV (OEL C) | 1920 mg/m³ |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 960 mg/m³ |
| OEL TWA | 500 ppm |
| OEL STEL | 1920 mg/m³ |
| OEL STEL | 1000 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-EC (OEL STEL) | 1910 mg/m³ |
| VLA-EC (OEL STEL) | 1000 ppm |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 1000 mg/m³ |
| NGV (OEL TWA) | 500 ppm |
| KGV (OEL STEL) | 1900 mg/m³ |
| KGV (OEL STEL) | 1000 ppm |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) | 1920 mg/m³ |
| WEL TWA (OEL TWA) | 1000 ppm |
| WEL STEL (OEL STEL) | 5760 mg/m³ (calculated) |
| WEL STEL (OEL STEL) | 3000 ppm (calculated) |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 950 mg/m³ |
| Grenseverdi (OEL TWA) | 500 ppm |
| Korttidsverdi (OEL STEL) | 1187.5 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) | 625 ppm (value calculated) |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 960 mg/m³ |
| MAK (OEL TWA) | 500 ppm |
| KZGW (OEL STEL) | 1920 mg/m³ |
| KZGW (OEL STEL) | 1000 ppm |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL STEL | 1000 ppm |
| ACGIH chemical category | Confirmed Animal Carcinogen with Unknown Relevance to Humans |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL STEL) | 42 mg/m³ |
| HTP (OEL STEL) | 10 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 40 mg/m³ |
| NDSCh (OEL STEL) | 80 mg/m³ |

| Carbitol (111-90-0) |  |
| --- | --- |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 35 mg/m³ |
| MAK (OEL TWA) | 6 ppm |
| MAK (OEL STEL) | 140 mg/m³ |
| MAK (OEL STEL) | 24 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 50.1 mg/m³ |
| OEL TWA | 10 ppm |
| OEL chemical category | skin notation |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 35 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) | 6 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 35 mg/m³ |
| OEL TWA | 6 ppm |
| OEL STEL | 70 mg/m³ |
| OEL STEL | 12 ppm |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 80 mg/m³ |
| NGV (OEL TWA) | 15 ppm |
| KGV (OEL STEL) | 170 mg/m³ |
| KGV (OEL STEL) | 30 ppm |
| OEL chemical category | skin notation |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 50 mg/m³ (aerosol, inhalable dust, vapour) |
| KZGW (OEL STEL) | 100 mg/m³ (aerosol, inhalable dust, vapour) |

8.1.2. Recommended monitoring procedures

No additional information available

8.1.3. Air contaminants formed

No additional information available

8.1.4. DNEL and PNEC

No additional information available

8.1.5. Control banding

No additional information available

8.2. Exposure controls

8.2.1. Appropriate engineering controls

No additional information available

8.2.2. Personal protection equipment

Personal protective equipment:

Avoid all unnecessary exposure.

Personal protective equipment symbol(s):

8.2.2.1. Eye and face protection

Eye protection:

Chemical goggles or safety glasses

8.2.2.2. Skin protection

Hand protection:

Wear protective gloves.

8.2.2.3. Respiratory protection

Respiratory protection:

Wear appropriate mask

8.2.2.4. Thermal hazards

No additional information available

8.2.3. Environmental exposure controls

Other information:

Do not eat, drink or smoke during use.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

| Physical state | : | Liquid |
| --- | --- | --- |
| Color | : | Standard. |
| Odor | : | characteristic. |
| Odor threshold | : | Not available |
| Melting point | : | Not available |
| Freezing point | : | Not available |
| Boiling point | : | Not available |
| Flammability | : | Non flammable. |
| Lower explosion limit | : | Not available |
| Upper explosion limit | : | Not available |
| Flash point | : | > 93 °C |
| Auto-ignition temperature | : | Not available |
| Decomposition temperature | : | Not available |
| pH | : | Not available |
| Viscosity, kinematic | : | Not available |
| Solubility | : | Not available |
| Partition coefficient n-octanol/water (Log Kow) | : | Not available |
| Vapor pressure | : | Not available |
| Vapor pressure at 50°C | : | Not available |
| Density | : | Not available |
| Relative density | : | Not available |
| Relative vapor density at 20°C | : | Not available |
| Particle characteristics | : | Not applicable |

9.2. Other information

9.2.1. Information with regard to physical hazard classes

No additional information available

9.2.2. Other safety characteristics

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

No additional information available

10.2. Chemical stability

Not established.

10.3. Possibility of hazardous reactions

Not established.

10.4. Conditions to avoid

Direct sunlight. Extremely high or low temperatures.

10.5. Incompatible materials

Strong acids. Strong bases.

10.6. Hazardous decomposition products

fume. Carbon monoxide. Carbon dioxide.

SECTION 11: Toxicological information

11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

| Acute toxicity (oral) | : | Not classified |
| --- | --- | --- |
| Acute toxicity (dermal) | : | Not classified |
| Acute toxicity (inhalation) | : | Not classified |

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| LD50 oral rat | 2450 mg/kg (Source: NLM_CIP) |
| LD50 oral | 2830 mg/kg body weight |
| LD50 dermal rabbit | > 7940 mg/kg (Source: NLM_CIP) |
| LC50 Inhalation - Rat (Dust/Mist) | 1.5 mg/l/4h |

| Hexyl cinnamic aldehyde (101-86-0) |  |
| --- | --- |
| LD50 oral rat | 3100 mg/kg (Source: NLM_CIP) |
| LD50 oral | 3100 mg/kg body weight |
| LD50 dermal rabbit | > 3000 mg/kg (Source: EPA_HPV) |
| LC50 Inhalation - Rat | > 5 mg/l/4h |

| Cyclamal (103-95-7) |  |
| --- | --- |
| LD50 oral rat | 3810 mg/kg (Source: NLM_CIP) |
| LD50 oral | 3810 mg/kg body weight |
| LD50 dermal rat | > 5000 mg/kg (Source: ECHA_API) |

| Melonal (106-72-9) |  |
| --- | --- |
| LD50 oral rat | > 5 g/kg (Source: NLM_CIP) |

| Hydroxy (107-75-5) |  |
| --- | --- |
| LD50 oral rat | > 5 g/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 2000 mg/kg (Source: ECHA_API) |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| LD50 oral rat | 4720 mg/kg (Source: NZ_CCID) |
| LD50 dermal rabbit | 3560 mg/kg (Source: NLM_CIP) |

| Helional (1205-17-0) |  |
| --- | --- |
| LD50 dermal rabbit | > 2000 mg/kg (Source: ECHA_API) |

| Amyl cinnamic aldehyde (122-40-7) |  |
| --- | --- |
| LD50 oral rat | 3730 mg/kg (Source: CHEMVIEW) |
| LD50 dermal rabbit | > 2000 mg/kg (Source: CHEMVIEW) |

| Methyl ionone (mixture of isomers) (1335-46-2) |  |
| --- | --- |
| LD50 oral rat | > 5000 mg/kg |
| LD50 dermal rabbit | > 5000 mg/kg (Source: CHEMVIEW) |
| LD50 dermal | 2900 mg/kg body weight |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| LD50 oral rat | 2490 mg/kg (Source: JAPAN_GHS) |
| LD50 oral | 2490 mg/kg body weight |
| LD50 dermal rabbit | > 5000 mg/kg (Source: JAPAN_GHS) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| LD50 oral rat | 3 g/kg (Source: NLM_HSDB) |
| LD50 oral | 4000 mg/kg body weight |
| LD50 dermal rabbit | 630 mg/kg (Source: NLM_HSDB) |

| Hexyl salicylate (6259-76-3) |  |
| --- | --- |
| LD50 oral rat | > 5 g/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 5000 mg/kg (Source: ECHA_API) |

| ethanol, ethyl alcohol (64-17-5) |  |
| --- | --- |
| LD50 oral rat | 7060 mg/kg (Source: NLM_CIP) |
| LC50 Inhalation - Rat | 133.8 mg/l/4h |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| LD50 oral rat | 4890 mg/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 8100 mg/kg (Source: ECHA_API) |

| Eugenol (97-53-0) |  |
| --- | --- |
| LD50 oral rat | 1930 mg/kg (Source: NZ_CCID) |
| LD50 oral | 2500 mg/kg body weight |

| isoeugenol (97-54-1) |  |
| --- | --- |
| LD50 oral rat | 1560 mg/kg (Source: NLM_CIP) |
| LD50 oral | 1500 mg/kg body weight |
| LD50 dermal | 1912 mg/kg body weight |

| Citronellol Pure (106-22-9) |  |
| --- | --- |
| LD50 oral rat | 3450 mg/kg (Source: NLM_CIP) |
| LD50 oral | 3450 mg/kg body weight |
| LD50 dermal rabbit | 2650 mg/kg (Source: EPA_HPV) |
| LD50 dermal | 2650 mg/kg body weight |

| Methyl heptine carbonate (111-12-6) |  |
| --- | --- |
| LD50 oral rat | 1530 mg/kg (Source: NLM_CIP) |

| Triplal (Vertocitral) (68039-49-6) |  |
| --- | --- |
| LD50 oral | 2330 mg/kg |

| 3-(2,2-dimethyl-3-hydroxypropyl)toluene, (alt.): 2,2-dimethyl-3-(3-methylphenyl)propanol (103694-68-4) |  |
| --- | --- |
| LD50 oral | 3440 mg/kg body weight |
| LD50 dermal rabbit | > 5 ml/kg (Source: ECHA_API) |

| Cyclogalbanate (Allyl Cyclohexyl Glycolate) (68901-15-5) |  |
| --- | --- |
| LD50 oral rat | 620 ml/kg |
| LD50 oral | 682 mg/kg body weight |
| LD50 dermal rat | > 2000 ml/kg |

| Linalool (78-70-6) |  |
| --- | --- |
| LD50 oral | 2790 mg/kg |

| benzyl benzoate (120-51-4) |  |
| --- | --- |
| LD50 oral rat | 500 mg/kg (Source: NLM_CIP) |
| LD50 oral | 1160 mg/kg body weight |
| LD50 dermal rabbit | 4000 mg/kg (Source: NLM_CIP) |

| Carbitol (111-90-0) |  |
| --- | --- |
| LD50 oral rat | 10502 mg/kg (Source: OECD_SIDS) |
| LD50 dermal rabbit | 9143 mg/kg (Source: OECD_SIDS) |
| LC50 Inhalation - Rat | > 5240 mg/m³ (Exposure time: 4 h Source: NLM_CIP) |

| Geraniol (106-24-1) |  |
| --- | --- |
| LD50 oral rat | 3600 mg/kg (Source: NLM_CIP) |
| LD50 oral | 3600 mg/kg body weight |
| LD50 dermal rabbit | > 5 g/kg (Source: NLM_CIP) |

| Nerol (106-25-2) |  |
| --- | --- |
| LD50 oral rat | 4500 mg/kg (Source: NLM_CIP) |
| LD50 oral | 4500 mg/kg body weight |
| LD50 dermal rabbit | > 5 g/kg (Source: NLM_CIP) |

| Skin corrosion/irritation | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Serious eye damage/irritation | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Respiratory or skin sensitization | : | May cause an allergic skin reaction. |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Germ cell mutagenicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Carcinogenicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| Eugenol (97-53-0) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| isoeugenol (97-54-1) |  |
| --- | --- |
| IARC group | 2B - Possibly carcinogenic to humans |

| Reproductive toxicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| STOT-single exposure | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| isoeugenol (97-54-1) |  |
| --- | --- |
| STOT-single exposure | May cause respiratory irritation. |

| STOT-repeated exposure | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Aspiration hazard | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| benzyl benzoate (120-51-4) |  |
| --- | --- |
| Viscosity, kinematic | 7.456 mm²/s |

11.2. Information on other hazards

11.2.1. Endocrine disrupting properties

No additional information available

11.2.2. Other information

| Potential Adverse human health effects and symptoms | : | Based on available data, the classification criteria are not met |
| --- | --- | --- |

SECTION 12: Ecological information

12.1. Toxicity

| Hazardous to the aquatic environment, short–term (acute) | : | Not classified |
| --- | --- | --- |
| Hazardous to the aquatic environment, long–term (chronic) | : | Harmful to aquatic life with long lasting effects. |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| LC50 - Fish [1] | 2.2 – 2.5 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 4.12 – 6.2 mg/l (Exposure time: 96 h - Species: Lepomis macrochirus [static] Source: EPA) |
| EC50 - Crustacea [1] | 3 mg/l (Exposure time: 48 h - Species: Daphnia magna) |

| Methyl ionone (mixture of isomers) (1335-46-2) |  |
| --- | --- |
| LC50 - Fish [1] | 2.3 mg/l (Exposure time: 96 h - Species: Danio rerio [static] Source: ECHA) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| LC50 - Fish [1] | 306 – 334 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 88 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static] Source: EPA) |

| ethanol, ethyl alcohol (64-17-5) |  |
| --- | --- |
| LC50 - Fish [2] | > 100 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static] Source: EPA) |
| EC50 - Crustacea [1] | 9268 – 14221 mg/l (Exposure time: 48 h - Species: Daphnia magna) |
| EC50 - Crustacea [2] | 2 mg/l (Exposure time: 48 h - Species: Daphnia magna [Static]) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| LC50 - Fish [1] | 12 – 16.5 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |

| Eugenol (97-53-0) |  |
| --- | --- |
| LC50 - Fish [1] | 13 mg/l (Exposure time: 96 h - Species: Danio rerio [semi-static] Source: ECHA) |

| Linalool (78-70-6) |  |
| --- | --- |
| EC50 96h - Algae [1] | 88.3 mg/l (Species: Desmodesmus subspicatus) |

| benzyl benzoate (120-51-4) |  |
| --- | --- |
| LC50 - Fish [1] | 2.32 mg/l (Exposure time: 96 h - Species: Danio rerio [semi-static] Source: ECHA) |
| NOEC (chronic) | 0.168 mg/l |

| Carbitol (111-90-0) |  |
| --- | --- |
| LC50 - Fish [1] | 10000 mg/l (Exposure time: 96 h - Species: Lepomis macrochirus [static] Source: EPA) |
| LC50 - Fish [2] | 19100 – 23900 mg/l (Exposure time: 96 h - Species: Lepomis macrochirus [flow-through] Source: EPA) |
| EC50 - Crustacea [1] | 3940 – 4670 mg/l (Exposure time: 48 h - Species: Daphnia magna) |

| Geraniol (106-24-1) |  |
| --- | --- |
| LC50 - Fish [1] | 22 mg/l (Exposure time: 96 h - Species: Danio rerio [static] Source: ECHA) |

| Nerol (106-25-2) |  |
| --- | --- |
| LC50 - Fish [1] | 20.3 mg/l (Exposure time: 96 h - Species: Danio rerio [semi-static] Source: ECHA) |

12.2. Persistence and degradability

| JAPANESE BAMBOO #EU47242F 25% in DPG |  |
| --- | --- |
| Persistence and degradability | Not established. |

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Hexyl cinnamic aldehyde (101-86-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Cyclamal (103-95-7) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Melonal (106-72-9) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Hydroxy (107-75-5) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Helional (1205-17-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Amyl cinnamic aldehyde (122-40-7) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Methyl ionone (mixture of isomers) (1335-46-2) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Hexyl salicylate (6259-76-3) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| ethanol, ethyl alcohol (64-17-5) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Eugenol (97-53-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| isoeugenol (97-54-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Citronellol Pure (106-22-9) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Methyl heptine carbonate (111-12-6) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Floralozone (67634-15-5) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Triplal (Vertocitral) (68039-49-6) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Bacdanol (28219-61-6) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| 3-(2,2-dimethyl-3-hydroxypropyl)toluene, (alt.): 2,2-dimethyl-3-(3-methylphenyl)propanol (103694-68-4) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Sandal Mysore Core (28219-60-5) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Cyclogalbanate (Allyl Cyclohexyl Glycolate) (68901-15-5) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Undecavertol (81782-77-6) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Linalool (78-70-6) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| benzyl benzoate (120-51-4) |  |
| --- | --- |
| Persistence and degradability | May cause long-term adverse effects in the environment. |

| Carbitol (111-90-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Geraniol (106-24-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Nerol (106-25-2) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

12.3. Bioaccumulative potential

| JAPANESE BAMBOO #EU47242F 25% in DPG |  |
| --- | --- |
| Bioaccumulative potential | Not established. |

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| BCF - Fish [1] | (470 dimensionless) |
| Partition coefficient n-octanol/water (Log Pow) | 4.21 (at 25 °C) |

| Cyclamal (103-95-7) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.4 (at 35 °C) |

| Melonal (106-72-9) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.4 (at 35 °C (at pH 7) |

| Hydroxy (107-75-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.68 (at 25 °C) |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.5 (at 25 °C (at pH 6) |

| Helional (1205-17-0) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.4 (at 25 °C) |

| Amyl cinnamic aldehyde (122-40-7) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.498 (at 25 °C (at pH 6.2) |

| Methyl ionone (mixture of isomers) (1335-46-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | (>4.5 - <5 - at 23 °C (at pH 6.2) |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.96 (at 25 °C (at pH 7) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.88 |

| Hexyl salicylate (6259-76-3) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 5.5 (at 30 °C (at pH 7) |

| ethanol, ethyl alcohol (64-17-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | -0.35 (at 24 °C (at pH 7.4) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.3 (at 25 °C (at pH 5) |

| Eugenol (97-53-0) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.83 (at 30 °C (at pH 5.5) |

| Citronellol Pure (106-22-9) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.41 (at 25 °C) |

| Methyl heptine carbonate (111-12-6) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3 (at 20 °C (at pH 7) |

| 3-(2,2-dimethyl-3-hydroxypropyl)toluene, (alt.): 2,2-dimethyl-3-(3-methylphenyl)propanol (103694-68-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.07 (at 20 °C) |

| Cyclogalbanate (Allyl Cyclohexyl Glycolate) (68901-15-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.8 (at 24.7 °C) |

| Undecavertol (81782-77-6) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.9 (at 30 °C (at pH 7) |

| benzyl benzoate (120-51-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.97 (at 25 °C) |
| Bioaccumulative potential | Not established. |

| Carbitol (111-90-0) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | -0.8 |

| Geraniol (106-24-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.6 (at 25 °C) |

| Nerol (106-25-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.76 (at 30 °C (at pH 6.5) |

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

No additional information available

12.6. Endocrine disrupting properties

No additional information available

12.7. Other adverse effects

| Additional information | : | Avoid release to the environment. |
| --- | --- | --- |

SECTION 13: Disposal considerations

13.1. Waste treatment methods

| Product/Packaging disposal recommendations | : | Dispose in a safe manner in accordance with local/national regulations. |
| --- | --- | --- |
| Ecological information | : | Avoid release to the environment. |
| HP code | : | HP4 - “Irritant – skin irritation and eye damage:” waste which on application can cause skin irritation or damage to the eye. HP14 - “Ecotoxic:” waste which presents or may present immediate or delayed risks for one or more sectors of the environment |

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number or ID number |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.2. UN proper shipping name |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.3. Transport hazard class(es) |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.4. Packing group |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.5. Environmental hazards |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| No supplementary information available |  |  |  |  |

14.6. Special precautions for user

Overland transport

Not applicable

Transport by sea

Not applicable

Air transport

Not applicable

Inland waterway transport

Not applicable

Rail transport

Not applicable

14.7. Maritime transport in bulk according to IMO instruments

Not applicable

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

15.1.1. EU-Regulations

REACH Annex XVII (Restriction List)

| EU restriction list (REACH Annex XVII) |  |  |
| --- | --- | --- |
| Reference code | Applicable on | Entry title or description |
| 3(a) | ethanol, ethyl alcohol ; Aldehyde C-6 | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 2.1 to 2.4, 2.6 and 2.7, 2.8 types A and B, 2.9, 2.10, 2.12, 2.13 categories 1 and 2, 2.14 categories 1 and 2, 2.15 types A to F |
| 3(b) | JAPANESE BAMBOO #EU47242F 25% in DPG ; Hexyl cinnamic aldehyde ; Cyclamal ; Melonal ; Hydroxy ; Helional ; Amyl cinnamic aldehyde ; Methyl ionone (mixture of isomers) ; Caproic acid ; Hexyl salicylate ; Eugenol ; isoeugenol ; Citronellol Pure ; Methyl heptine carbonate ; Floralozone ; Triplal (Vertocitral) ; Bacdanol ; 3-(2,2-dimethyl-3-hydroxypropyl)toluene, (alt.): 2,2-dimethyl-3-(3-methylphenyl)propanol ; Sandal Mysore Core ; Cyclogalbanate (Allyl Cyclohexyl Glycolate) ; Linalool ; benzyl benzoate ; Geraniol ; Nerol | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 3.1 to 3.6, 3.7 adverse effects on sexual function and fertility or on development, 3.8 effects other than narcotic effects, 3.9 and 3.10 |
| 3(c) | JAPANESE BAMBOO #EU47242F 25% in DPG ; Hexyl cinnamic aldehyde ; Cyclamal ; Alcohol C-10 ; Helional ; Amyl cinnamic aldehyde ; Methyl ionone (mixture of isomers) ; Benzyl acetate ; Hexyl salicylate ; Methyl heptine carbonate ; Floralozone ; Triplal (Vertocitral) ; Bacdanol ; 3-(2,2-dimethyl-3-hydroxypropyl)toluene, (alt.): 2,2-dimethyl-3-(3-methylphenyl)propanol ; Sandal Mysore Core ; Cyclogalbanate (Allyl Cyclohexyl Glycolate) ; Undecavertol ; benzyl benzoate | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard class 4.1 |
| 40. | ethanol, ethyl alcohol ; Aldehyde C-6 | Substances classified as flammable gases category 1 or 2, flammable liquids categories 1, 2 or 3, flammable solids category 1 or 2, substances and mixtures which, in contact with water, emit flammable gases, category 1, 2 or 3, pyrophoric liquids category 1 or pyrophoric solids category 1, regardless of whether they appear in Part 3 of Annex VI to Regulation (EC) No 1272/2008 or not. |

REACH Annex XIV (Authorisation List)

Contains no REACH Annex XIV substances.

REACH Candidate List (SVHC)

Contains no REACH candidate substance

PIC Regulation (Prior Informed Consent)

Contains no substance subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 July 2012 concerning the export and import of hazardous chemicals.

POP Regulation (Persistent Organic Pollutants)

Contains no substance(s) subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

Ozone Regulation (1005/2009)

Contains no substance subject to REGULATION (EU) No 1005/2009 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 September 2009 on substances that deplete the ozone layer.

Dual-Use Regulation (428/2009)

Contains no substance subject to the COUNCIL REGULATION (EC) No 428/2009 of 5 May 2009 setting up a Community regime for the control of exports, transfer, brokering and transit of dual-use items.

Explosives Precursors Regulation (2019/1148)

Contains no substance subject to Regulation (EU) 2019/1148 of the European Parliament and of the Council of 20 June 2019 on the marketing and use of explosives precursors.

Drug Precursors Regulation (273/2004)

Contains no substance subject to Regulation (EC) 273/2004 of the European Parliament and of the Council of 11 February 2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances.

15.1.2. National regulations

| France |  |  |
| --- | --- | --- |
| Professional diseases. Code; Description. RG 84; Conditions caused by liquid organic solvents for professional use: saturated or unsaturated aliphatic or cyclic liquid hydrocarbons and mixtures thereof; liquid halogenated hydrocarbons; nitrated derivatives of aliphatic hydrocarbons; alcohols; glycols, glycol ethers; ketones; aldehydes; aliphatic and cyclic ethers, including tetrahydrofuran; esters; dimethylformamide and dimethylacetamine; acetonitrile and propionitrile; pyridine; dimethylsulfone and dimethylsulfoxide. |
| Germany |  |  |
| Water hazard class (WGK) | : | WGK 2, significant hazardous to water (Classification according to AwSV, Annex 1). |
| Hazardous Incident Ordinance (12. BImSchV) | : | Is not subject to the Hazardous Incident Ordinance (12. BImSchV) |
| Netherlands |  |  |
| ABM category | : | A(2) - toxic for aquatic organisms, may have longterm hazardous effects in aquatic environment |
| SZW-lijst van kankerverwekkende stoffen | : | Ethyl alcohol,Floralozone,Triplal (Vertocitral),Cyclogalbanate (Allyl Cyclohexyl Glycolate) are listed |
| SZW-lijst van mutagene stoffen | : | Floralozone,Triplal (Vertocitral),Cyclogalbanate (Allyl Cyclohexyl Glycolate) are listed |
| SZW-lijst van reprotoxische stoffen – Borstvoeding | : | Ethyl alcohol is listed |
| SZW-lijst van reprotoxische stoffen – Vruchtbaarheid | : | Ethyl alcohol is listed |
| SZW-lijst van reprotoxische stoffen – Ontwikkeling | : | Ethyl alcohol is listed |
| Denmark |  |  |
| Classification remarks | : | Emergency management guidelines for the storage of flammable liquids must be followed |
| Danish National Regulations | : | Young people below the age of 18 years are not allowed to use the product Pregnant/breastfeeding women working with the product must not be in direct contact with the product |

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

| Data sources | : | REGULATION (EC) No 1272/2008 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 December 2008 on classification, labeling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006. |
| --- | --- | --- |
| Other information | : | None. |

| Full text of H- and EUH-phrases: |  |
| --- | --- |
| Acute Tox. 4 (Dermal) | Acute toxicity (dermal), Category 4 |
| Acute Tox. 4 (Inhalation) | Acute toxicity (inhalation) Category 4 |
| Acute Tox. 4 (Oral) | Acute toxicity (oral) Category 4 |
| Aquatic Acute 1 | Hazardous to the aquatic environment – Acute Hazard Category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment – Chronic Hazard Category 1 |
| Aquatic Chronic 2 | Hazardous to the aquatic environment – Chronic Hazard Category 2 |
| Aquatic Chronic 3 | Hazardous to the aquatic environment – Chronic Hazard Category 3 |
| Carc. 2 | Carcinogenicity Category 2 |
| Eye Dam. 1 | Serious eye damage/eye irritation Category 1 |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 2 | Flammable liquids Category 2 |
| Flam. Liq. 3 | Flammable liquids Category 3 |
| H225 | Highly flammable liquid and vapor. |
| H226 | Flammable liquid and vapor. |
| H302 | Harmful if swallowed. |
| H312 | Harmful in contact with skin. |
| H314 | Causes severe skin burns and eye damage. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H318 | Causes serious eye damage. |
| H319 | Causes serious eye irritation. |
| H332 | Harmful if inhaled. |
| H335 | May cause respiratory irritation. |
| H351 | Suspected of causing cancer. |
| H361 | Suspected of damaging fertility or the unborn child. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H411 | Toxic to aquatic life with long lasting effects. |
| H412 | Harmful to aquatic life with long lasting effects. |
| Repr. 2 | Reproductive toxicity Category 2 |
| Skin Corr. 1C | Skin corrosion/irritation, Category 1, Sub-Category 1C |
| Skin Irrit. 2 | Skin corrosion/irritation Category 2 |
| Skin Sens. 1 | Skin sensitization, Category 1 |
| Skin Sens. 1A | Skin sensitization, Category 1A |
| Skin Sens. 1B | Skin sensitization, Category 1B |
| STOT SE 3 | Specific target organ toxicity – Single exposure, Category 3, Respiratory tract irritation |

| The classification complies with | : | ATP 12 |
| --- | --- | --- |

Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.