SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

| Product form | : | Mixture |
| --- | --- | --- |
| Product name | : | ACAI BERRY BOOST #EU55090F 25% |
| Product code | : | EU55090F_25% |
| Type of product | : | Perfumes, Fragrances |

1.2. Relevant identified uses of the substance or mixture and uses advised against

1.2.1. Relevant identified uses

| Industrial/Professional use spec | : | Industrial For professional use only |
| --- | --- | --- |
| Use of the substance/mixture | : | Perfumes, Fragrances |
| Function or use category | : | Odour agents |

1.2.2. Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

No additional information available

1.4. Emergency telephone number

No additional information available

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

| Skin sensitization, Category 1 | H317 |  |
| --- | --- | --- |
| Hazardous to the aquatic environment – Chronic Hazard Category 2 | H411 |  |

Full text of H- and EUH-statements: see section 16

Adverse physicochemical, human health and environmental effects

No additional information available

2.2. Label elements

Labeling according to Regulation (EC) No. 1272/2008 [CLP]

| Hazard pictograms (CLP) | : |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GHS07 | GHS09 |  |  |  |  |
| Signal word (CLP) | : | Warning |  |  |  |  |  |
| Contains | : | Hexyl salicylate; Iso E Super; Hexyl cinnamic aldehyde; Linalool; CUPRESSUS FUNEBRIS WOOD OIL; Linalyl acetate; d-Limonene; Citral; Orange oil ; Helional; Geraniol; Aldehyde C-16; Triplal (Vertocitral); Patchouli oil |  |  |  |  |  |
| Hazard statements (CLP) | : | H317 - May cause an allergic skin reaction. H411 - Toxic to aquatic life with long lasting effects. |  |  |  |  |  |
| Precautionary statements (CLP) | : | P261 - Avoid breathing dust/fume/gas/mist/vapors/spray. P272 - Contaminated work clothing should not be allowed out of the workplace. P273 - Avoid release to the environment. P280 - Wear protective gloves/protective clothing/eye protection/face protection/hearing protection. P302+P352 - IF ON SKIN: Wash with plenty of water. P321 - Specific treatment (see supplemental first aid instruction on this label). |  |  |  |  |  |
| Extra phrases | : | Restricted to professional users. |  |  |  |  |  |

2.3. Other hazards

Contains no PBT/vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII

SECTION 3: Composition/Information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| 2-Isobutyl-4-methyltetrahydro-2H-pyran-4-ol | CAS-No.: 63500-71-0 EC-No.: 405-040-6 EC Index-No.: 603-101-00-3 REACH-no: 01-000015458-64 | 0.5 – 1.903 | Eye Irrit. 2, H319 |
| Benzyl acetate substance with national workplace exposure limit(s) (BE, DK, ES, IE, LT, LV, PT, RO) | CAS-No.: 140-11-4 EC-No.: 205-399-7 REACH-no: 01-2119638272-42 | 0.95 – 1.875 | Aquatic Chronic 3, H412 |
| Iso E Super | CAS-No.: 54464-57-2 EC-No.: 259-174-3 REACH-no: 01-2119489989-04 | 0.825 – 1.625 | Skin Irrit. 2, H315 Skin Sens. 1, H317 Aquatic Chronic 1, H410 |
| Hexyl cinnamic aldehyde | CAS-No.: 101-86-0 EC-No.: 202-983-3 REACH-no: 01-2119533092-50 | 0.625 – 1.25 | Skin Sens. 1, H317 Aquatic Chronic 2, H411 |
| Linalool | CAS-No.: 78-70-6 EC-No.: 201-134-4 EC Index-No.: 603-235-00-2 REACH-no: 01-2119474016-42 | 0.5 – 0.975 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| CUPRESSUS FUNEBRIS WOOD OIL | CAS-No.: 85085-29-6 EC-No.: 285-360-9 | 0.45 – 0.9 | Skin Irrit. 1, H314 Skin Sens. 1, H317 Asp. Tox. 1, H304 Aquatic Chronic 2, H411 |
| Hexamethylindanopyran | CAS-No.: 1222-05-5 EC-No.: 214-946-9 EC Index-No.: 603-212-00-7 REACH-no: 01-2119488227-29 | 0.325 – 0.625 | Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| Linalyl acetate | CAS-No.: 115-95-7 EC-No.: 204-116-4 REACH-no: 01-2119454789-19 | 0.3 – 0.6 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1, H317 |
| d-Limonene substance with national workplace exposure limit(s) (DE, ES, FI, SI, NO, CH) | CAS-No.: 5989-27-5 EC-No.: 205-341-0 EC Index-No.: 601-096-00-2 REACH-no: 01-2119493353-35 | 0.275 – 0.55 | Flam. Liq. 3, H226 Skin Irrit. 2, H315 Skin Sens. 1B, H317 Asp. Tox. 1, H304 Aquatic Acute 1, H400 Aquatic Chronic 3, H412 |
| Citral substance with national workplace exposure limit(s) (BE, ES, IE, PL, PT) | CAS-No.: 5392-40-5 EC-No.: 226-394-6 EC Index-No.: 605-019-00-3 REACH-no: 01-2119462829-23 | 0.25 – 0.5 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1, H317 |
| Hexyl salicylate | CAS-No.: 6259-76-3 EC-No.: 228-408-6 | 0.1095 – 0.385875 | Skin Irrit. 2, H315 Skin Sens. 1, H317 Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| Orange oil | CAS-No.: 8008-57-9 EC-No.: 232-433-8 REACH-no: 01-2119493353-35 | 0.2 – 0.375 | Flam. Liq. 3, H226 Skin Irrit. 2, H315 Skin Sens. 1, H317 Asp. Tox. 1, H304 Aquatic Acute 1, H400 Aquatic Chronic 2, H411 |
| Helional | CAS-No.: 1205-17-0 EC-No.: 214-881-6 REACH-no: 01-2120740119-58 | 0.15 – 0.275 | Skin Sens. 1B, H317 Repr. 2, H361 Aquatic Chronic 2, H411 |
| Aldehyde C-16 | CAS-No.: 77-83-8 EC-No.: 201-061-8 REACH-no: 01-2119967770-28 | 0.1 – 0.2 | Skin Sens. 1B, H317 Aquatic Chronic 2, H411 |
| Benzyl benzoate | CAS-No.: 120-51-4 EC-No.: 204-402-9 EC Index-No.: 607-085-00-9 REACH-no: 01-2119976371-33 | 0.1 – 0.176 | Acute Tox. 4 (Oral), H302 Aquatic Acute 1, H400 Aquatic Chronic 2, H411 |
| Triplal (Vertocitral) | CAS-No.: 68039-49-6 EC-No.: 268-264-1 | 0.075 – 0.1269375 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1, H317 Aquatic Chronic 3, H412 |
| Patchouli oil | CAS-No.: 8014-09-3 EC Index-No.: 616-944-7 | 0.075 – 0.125 | Skin Sens. 1B, H317 Asp. Tox. 1, H304 Aquatic Chronic 2, H411 |
| Allyl caproate | CAS-No.: 123-68-2 EC-No.: 204-642-4 REACH-no: 01-2119983573-26 | 0.075 – 0.125 | Acute Tox. 3 (Oral), H301 Acute Tox. 3 (Dermal), H311 Acute Tox. 3 (Inhalation), H331 Aquatic Acute 1, H400 Aquatic Chronic 3, H412 |
| Geraniol | CAS-No.: 106-24-1 EC-No.: 203-377-1 EC Index-No.: 603-241-00-5 REACH-no: 01-2119552430-49 | 0.06 – 0.12 | Skin Irrit. 2, H315 Eye Dam. 1, H318 Skin Sens. 1, H317 |
| Ethyl alcohol substance with national workplace exposure limit(s) (AT, BE, BG, CZ, DE, DK, EE, ES, FI, FR, GB, GR, HR, HU, IE, LT, LV, NL, PL, PT, RO, SE, SI, SK, NO, CH) | CAS-No.: 64-17-5 EC-No.: 200-578-6 EC Index-No.: 603-002-00-5 | 0.025 – 0.062375 | Flam. Liq. 2, H225 |
| 1,2-Propanediol substance with national workplace exposure limit(s) (GB, HR, IE, LT, LV, PL, NO) | CAS-No.: 57-55-6 EC-No.: 200-338-0 REACH-no: 01-2119456809-23 | 0.025 – 0.06175 | Not classified |
| decyl alcohol substance with national workplace exposure limit(s) (BG, DE, LT, LV, RO, CH) | CAS-No.: 112-30-1 EC-No.: 203-956-9 | 0 – 0.0112 | Aquatic Chronic 3, H412 |
| .beta.-Pinene substance with national workplace exposure limit(s) (BE, EE, ES, LT, PT, SE, NO) | CAS-No.: 127-91-3 EC-No.: 204-872-5 | ≤ 0.005625 | Flam. Liq. 3, H226 |
| Dipropylene glycol monomethyl ether substance with national workplace exposure limit(s) (AT, BE, BG, CY, CZ, DE, DK, EE, ES, FI, FR, GB, GI, GR, HR, HU, IE, IT, LT, LU, LV, MT, NL, PL, PT, RO, SE, SI, SK, NO, CH, TR); substance with a Community workplace exposure limit | CAS-No.: 34590-94-8 EC-No.: 252-104-2 | ≤ 0.00465 | Not classified |
| Aldehyde C-6 substance with national workplace exposure limit(s) (FI, PL) | CAS-No.: 66-25-1 EC-No.: 200-624-5 | 0 – 0.0028 | Flam. Liq. 3, H226 |
| .alpha.-Pinene substance with national workplace exposure limit(s) (BE, EE, ES, LT, PT, SE, NO) | CAS-No.: 80-56-8 EC-No.: 201-291-9 | ≤ 0.000375 | Flam. Liq. 3, H226 |
| Caproic acid substance with national workplace exposure limit(s) (BG, LT, LV) | CAS-No.: 142-62-1 EC-No.: 205-550-7 | 0 – 0.00015 | Eye Dam. 1, H318 Skin Corr. 1C, H314 |
| CAFFEINE substance with national workplace exposure limit(s) (BG, LT, LV) | CAS-No.: 58-08-2 EC-No.: 200-362-1 EC Index-No.: 613-086-00-5 | 0 – 0.000075 | Acute Tox. 4 (Oral), H302 |
| Butyric acid substance with national workplace exposure limit(s) (BG, LT, LV, RO) | CAS-No.: 107-92-6 EC-No.: 203-532-3 EC Index-No.: 607-135-00-X | 0 – 0.00005 | Skin Corr. 1B, H314 |

Full text of H- and EUH-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

| First-aid measures general | : | Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label where possible). |
| --- | --- | --- |
| First-aid measures after inhalation | : | Allow affected person to breathe fresh air. Allow the victim to rest. |
| First-aid measures after skin contact | : | Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse. |
| First-aid measures after eye contact | : | Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persists. |
| First-aid measures after ingestion | : | Rinse mouth. Do NOT induce vomiting. Obtain emergency medical attention. |

4.2. Most important symptoms and effects, both acute and delayed

| Symptoms/effects | : | Not expected to present a significant hazard under anticipated conditions of normal use. |
| --- | --- | --- |

4.3. Indication of any immediate medical attention and special treatment needed

No additional information available

SECTION 5: Firefighting measures

5.1. Extinguishing media

| Suitable extinguishing media | : | Foam. Dry powder. Carbon dioxide. Water spray. Sand. |
| --- | --- | --- |
| Unsuitable extinguishing media | : | Do not use a heavy water stream. |

5.2. Special hazards arising from the substance or mixture

No additional information available

5.3. Advice for firefighters

| Firefighting instructions | : | Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. Prevent fire-fighting water from entering environment. |
| --- | --- | --- |
| Protection during firefighting | : | Do not enter fire area without proper protective equipment, including respiratory protection. |

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

6.1.1. For non-emergency personnel

| Emergency procedures | : | Evacuate unnecessary personnel. |
| --- | --- | --- |

6.1.2. For emergency responders

| Protective equipment | : | Equip cleanup crew with proper protection. |
| --- | --- | --- |
| Emergency procedures | : | Ventilate area. |

6.2. Environmental precautions

Prevent entry to sewers and public waters. Notify authorities if liquid enters sewers or public waters.

6.3. Methods and material for containment and cleaning up

| Methods for cleaning up | : | Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Collect spillage. Store away from other materials. |
| --- | --- | --- |

6.4. Reference to other sections

See Heading 8. Exposure controls and personal protection.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

| Precautions for safe handling | : | Wash hands and other exposed areas with mild soap and water before eating, drinking or smoking and when leaving work. Provide good ventilation in process area to prevent formation of vapor. |
| --- | --- | --- |

7.2. Conditions for safe storage, including any incompatibilities

| Storage conditions | : | Keep only in the original container in a cool, well ventilated place away from : Keep container closed when not in use. |
| --- | --- | --- |
| Incompatible products | : | Strong bases. Strong acids. |
| Incompatible materials | : | Sources of ignition. Direct sunlight. |

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

8.1.1 National occupational exposure and biological limit values

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 62 mg/m³ |
| OEL TWA [ppm] | 10 ppm |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA [1] | 61 mg/m³ |
| OEL TWA [2] | 10 ppm |
| OEL STEL | 122 mg/m³ |
| OEL STEL [ppm] | 20 ppm |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA [2] | 10 ppm |
| OEL STEL [ppm] | 30 ppm (calculated) |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 5 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA [ppm] | 10 ppm |
| OEL chemical category | A4 - Not Classifiable as a Human Carcinogen |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 50 mg/m³ |
| OEL TWA [ppm] | 8 ppm |
| OEL STEL | 80 mg/m³ |
| OEL STEL [ppm] | 13 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) [1] | 62 mg/m³ |
| VLA-ED (OEL TWA) [2] | 10 ppm |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA [ppm] | 10 ppm |
| ACGIH chemical category | Not Classifiable as a Human Carcinogen |

| d-Limonene (5989-27-5) |  |
| --- | --- |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) [1] | 140 mg/m³ |
| HTP (OEL TWA) [2] | 25 ppm |
| HTP (OEL STEL) | 280 mg/m³ |
| HTP (OEL STEL) [ppm] | 50 ppm |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) [1] | 28 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) [2] | 5 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Chemical category | skin notation, Skin sensitization |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 28 mg/m³ |
| OEL TWA [ppm] | 5 ppm |
| OEL STEL | 112 mg/m³ |
| OEL STEL [ppm] | 20 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) [1] | 168 mg/m³ |
| VLA-ED (OEL TWA) [2] | 30 ppm |
| OEL chemical category | Sensitizer, skin - potential for cutaneous absorption |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) [1] | 140 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 25 ppm |
| Korttidsverdi (OEL STEL) | 175 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) [ppm] | 37.5 ppm (value calculated) |
| OEL chemical category | Allergenic substance |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) [1] | 40 mg/m³ |
| MAK (OEL TWA) [2] | 7 ppm |
| KZGW (OEL STEL) | 80 mg/m³ |
| KZGW (OEL STEL) [ppm] | 14 ppm |
| OEL chemical category | Sensitizer |

| Citral (5392-40-5) |  |
| --- | --- |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 32 mg/m³ (vapor and aerosol) |
| OEL TWA [ppm] | 5 ppm (vapor and aerosol) |
| OEL chemical category | Skin |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA [2] | 5 ppm |
| OEL STEL [ppm] | 15 ppm (calculated) |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 27 mg/m³ |
| NDSCh (OEL STEL) | 54 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA [ppm] | 5 ppm (inhalable fraction; vapor) |
| OEL chemical category | Sensitizer dermal, A4 - Not Classifiable as a Human Carcinogen, skin - potential for cutaneous exposure |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) [2] | 5 ppm (inhalable fraction and vapor) |
| OEL chemical category | Sensitizer, skin - potential for cutaneous absorption |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA [ppm] | 5 ppm (inhalable fraction and vapor) |
| ACGIH chemical category | Not Classifiable as a Human Carcinogen, Skin - potential significant contribution to overall exposure by the cutaneous route, dermal sensitizer |

| Ethyl alcohol (64-17-5) |  |
| --- | --- |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 1900 mg/m³ |
| MAK (OEL TWA) [ppm] | 1000 ppm |
| MAK (OEL STEL) | 3800 mg/m³ |
| MAK (OEL STEL) [ppm] | 2000 ppm |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 1907 mg/m³ |
| OEL TWA [ppm] | 1000 ppm |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 1000 mg/m³ |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) [1] | 1900 mg/m³ |
| GVI (OEL TWA) [2] | 1000 ppm |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 1000 mg/m³ |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA [1] | 1900 mg/m³ |
| OEL TWA [2] | 1000 ppm |
| OEL STEL | 3800 mg/m³ |
| OEL STEL [ppm] | 2000 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 1000 mg/m³ |
| OEL TWA [ppm] | 500 ppm |
| OEL STEL | 1900 mg/m³ |
| OEL STEL [ppm] | 1000 ppm |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) [1] | 1900 mg/m³ |
| HTP (OEL TWA) [2] | 1000 ppm |
| HTP (OEL STEL) | 2500 mg/m³ |
| HTP (OEL STEL) [ppm] | 1300 ppm |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 1900 mg/m³ |
| VME (OEL TWA) [ppm] | 1000 ppm |
| VLE (OEL C/STEL) | 9500 mg/m³ |
| VLE (OEL C/STEL) [ppm] | 5000 ppm |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) [1] | 380 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) [2] | 200 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 1900 mg/m³ |
| OEL TWA [ppm] | 1000 ppm |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 1900 mg/m³ |
| CK (OEL STEL) | 3800 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL STEL [ppm] | 1000 ppm |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 1000 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 1000 mg/m³ |
| IPRV (OEL TWA) [ppm] | 500 ppm |
| TPRV (OEL STEL) | 1900 mg/m³ |
| TPRV (OEL STEL) [ppm] | 1000 ppm |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 260 mg/m³ |
| TGG-8u (OEL TWA) [ppm] | 137 ppm |
| TGG-15min (OEL STEL) | 1900 mg/m³ |
| TGG-15min (OEL STEL) [ppm] | 1000 ppm |
| MAC chemical category | skin notation |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 1900 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL STEL [ppm] | 1000 ppm |
| OEL chemical category | A3 - Confirmed Animal Carcinogen with Unknown Relevance to Humans |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 1900 mg/m³ |
| OEL TWA [ppm] | 1000 ppm |
| OEL STEL | 9500 mg/m³ |
| OEL STEL [ppm] | 5000 ppm |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) [1] | 960 mg/m³ |
| NPHV (OEL TWA) [2] | 500 ppm |
| NPHV (OEL C) | 1920 mg/m³ |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 960 mg/m³ |
| OEL TWA [ppm] | 500 ppm |
| OEL STEL | 1920 mg/m³ |
| OEL STEL [ppm] | 1000 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-EC (OEL STEL) | 1910 mg/m³ |
| VLA-EC (OEL STEL) [ppm] | 1000 ppm |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 1000 mg/m³ |
| NGV (OEL TWA) [ppm] | 500 ppm |
| KTV (OEL STEL) | 1900 mg/m³ |
| KTV (OEL STEL) [ppm] | 1000 ppm |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) [1] | 1920 mg/m³ |
| WEL TWA (OEL TWA) [2] | 1000 ppm |
| WEL STEL (OEL STEL) | 5760 mg/m³ (calculated) |
| WEL STEL (OEL STEL) [ppm] | 3000 ppm (calculated) |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) [1] | 950 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 500 ppm |
| Korttidsverdi (OEL STEL) | 1187.5 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) [ppm] | 625 ppm (value calculated) |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) [1] | 960 mg/m³ |
| MAK (OEL TWA) [2] | 500 ppm |
| KZGW (OEL STEL) | 1920 mg/m³ |
| KZGW (OEL STEL) [ppm] | 1000 ppm |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL STEL [ppm] | 1000 ppm |
| ACGIH chemical category | Confirmed Animal Carcinogen with Unknown Relevance to Humans |

| 1,2-Propanediol (57-55-6) |  |
| --- | --- |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) [1] | 474 mg/m³ (total vapor and particles) 10 mg/m³ (particles) |
| GVI (OEL TWA) [2] | 150 ppm |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA [1] | 10 mg/m³ (particulates) 470 mg/m³ (total vapour and particulates) |
| OEL TWA [2] | 150 ppm (total vapour and particulates) |
| OEL STEL | 1410 mg/m³ (calculated-particulates) 30 mg/m³ (calculated) |
| OEL STEL [ppm] | 450 ppm (calculated-total vapour and particulates) |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 7 mg/m³ |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 100 mg/m³ (vapor and inhalable fraction) |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) [1] | 474 mg/m³ (total vapour and particulates) 10 mg/m³ (particulates) |
| WEL TWA (OEL TWA) [2] | 150 ppm (total vapour and particulates) |
| WEL STEL (OEL STEL) | 1422 mg/m³ (calculated-total vapour and particulates) 30 mg/m³ (calculated-particulate) |
| WEL STEL (OEL STEL) [ppm] | 450 ppm (calculated-total vapour and particulates) |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) [1] | 79 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 25 ppm |
| Korttidsverdi (OEL STEL) | 118.5 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) [ppm] | 37.5 ppm (value calculated) |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| EU - Indicative Occupational Exposure Limit (IOEL) |  |
| IOEL TWA | 308 mg/m³ |
| IOEL TWA [ppm] | 50 ppm |
| Remark | Possibility of significant uptake through the skin |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 307 mg/m³ (mixed isomers) |
| MAK (OEL TWA) [ppm] | 50 ppm (mixed isomers) |
| MAK (OEL STEL) | 614 mg/m³ (isomers mixtures) |
| MAK (OEL STEL) [ppm] | 100 ppm (isomers mixtures) |
| OEL chemical category | skin notation |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | Skin, skin notation |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) [1] | 308 mg/m³ |
| GVI (OEL TWA) [2] | 50 ppm |
| OEL chemical category | skin notation |
| Cyprus - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | Skin-potential for cutaneous absorption |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 270 mg/m³ |
| OEL chemical category | Potential for cutaneous absorption |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA [1] | 309 mg/m³ |
| OEL TWA [2] | 50 ppm |
| OEL STEL | 618 mg/m³ |
| OEL STEL [ppm] | 100 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | skin notation |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) [1] | 310 mg/m³ |
| HTP (OEL TWA) [2] | 50 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 308 mg/m³ (restrictive limit) |
| VME (OEL TWA) [ppm] | 50 ppm (restrictive limit) |
| OEL chemical category | Risk of cutaneous absorption |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) [1] | 310 mg/m³ (isomer mixture) |
| AGW (OEL TWA) [2] | 50 ppm (isomer mixture) |
| Gibraltar - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | skin notation |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 600 mg/m³ |
| OEL TWA [ppm] | 100 ppm |
| OEL STEL | 900 mg/m³ |
| OEL STEL [ppm] | 150 ppm |
| OEL chemical category | skin - potential for cutaneous absorption |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 308 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA [1] | 308 mg/m³ ((2-Methoxymethylethoxy)propanol) |
| OEL TWA [2] | 50 ppm ((2-Methoxymethylethoxy)propanol) |
| OEL STEL | 924 mg/m³ (calculated (2-(2-Methoxypropoxy)-1-propanol) |
| OEL STEL [ppm] | 150 ppm (calculated (2-(2-Methoxypropoxy)-1-propanol) |
| OEL chemical category | Potential for cutaneous absorption |
| Italy - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | skin - potential for cutaneous absorption |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | skin - potential for cutaneous exposure |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 300 mg/m³ (2-(2-Methoxypropoxy)-propanol) |
| IPRV (OEL TWA) [ppm] | 50 ppm (2-(2-Methoxypropoxy)-propanol) |
| TPRV (OEL STEL) | 450 mg/m³ (2-(2-Methoxypropoxy)-propanol) |
| TPRV (OEL STEL) [ppm] | 75 ppm (2-(2-Methoxypropoxy)-propanol) |
| OEL chemical category | skin notation |
| Luxembourg - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | Possibility of significant uptake through the skin |
| Malta - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | Possibility of significant uptake through the skin |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 300 mg/m³ |
| TGG-8u (OEL TWA) [ppm] | 48.7 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 240 mg/m³ (mixture of isomers: 1-(2-Methoxy-1-methylethoxy)propan-2-ol, 1-(2-Methoxy-2-methylethoxy)propan-2-ol and 2-(2-Methoxy-1-methylethoxy)propan-1-ol) |
| NDSCh (OEL STEL) | 480 mg/m³ (mixture of isomers: 1-(2-Methoxy-1-methylethoxy)propan-2-ol, 1-(2-Methoxy-2-methylethoxy)propan-2-ol, 2-(2-Methoxy-1-methylethoxy)propan-1-ol) |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ (indicative limit value) |
| OEL TWA [ppm] | 50 ppm (indicative limit value) |
| OEL STEL [ppm] | 150 ppm |
| OEL chemical category | skin - potential for cutaneous exposure indicative limit value |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL chemical category | skin notation |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) [1] | 308 mg/m³ |
| NPHV (OEL TWA) [2] | 50 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA [ppm] | 50 ppm |
| OEL STEL | 308 mg/m³ |
| OEL STEL [ppm] | 50 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) [1] | 308 mg/m³ (indicative limit value) |
| VLA-ED (OEL TWA) [2] | 50 ppm (indicative limit value) |
| OEL chemical category | skin - potential for cutaneous absorption |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 300 mg/m³ |
| NGV (OEL TWA) [ppm] | 50 ppm |
| KTV (OEL STEL) | 450 mg/m³ |
| KTV (OEL STEL) [ppm] | 75 ppm |
| OEL chemical category | skin notation |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) [1] | 308 mg/m³ |
| WEL TWA (OEL TWA) [2] | 50 ppm |
| WEL STEL (OEL STEL) | 924 mg/m³ (calculated) |
| WEL STEL (OEL STEL) [ppm] | 150 ppm (calculated) |
| WEL chemical category | Potential for cutaneous absorption |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) [1] | 300 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 50 ppm |
| Korttidsverdi (OEL STEL) | 375 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) [ppm] | 75 ppm (value calculated) |
| OEL chemical category | skin notation |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) [1] | 300 mg/m³ (aerosol, vapour) |
| MAK (OEL TWA) [2] | 50 ppm (aerosol, vapour) |
| KZGW (OEL STEL) | 300 mg/m³ (aerosol, vapour) |
| KZGW (OEL STEL) [ppm] | 50 ppm (aerosol, vapour) |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA [ppm] | 50 ppm (Dipropylene glycol methyl ether) |

| .alpha.-Pinene (80-56-8) |  |
| --- | --- |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA [ppm] | 20 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 150 mg/m³ (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| OEL TWA [ppm] | 25 ppm (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| OEL STEL | 300 mg/m³ (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| OEL STEL [ppm] | 50 ppm (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 150 mg/m³ |
| IPRV (OEL TWA) [ppm] | 25 ppm |
| TPRV (OEL STEL) | 300 mg/m³ |
| TPRV (OEL STEL) [ppm] | 50 ppm |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA [ppm] | 20 ppm (Turpentine and selected Monoterpenes) |
| OEL chemical category | Sensitizer dermal, A4 - Not Classifiable as a Human Carcinogen |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) [1] | 113 mg/m³ |
| VLA-ED (OEL TWA) [2] | 20 ppm |
| OEL chemical category | Sensitizer |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 150 mg/m³ |
| NGV (OEL TWA) [ppm] | 25 ppm |
| KTV (OEL STEL) | 300 mg/m³ |
| KTV (OEL STEL) [ppm] | 50 ppm |
| OEL chemical category | Sensitizer |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) [1] | 140 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 25 ppm |
| Korttidsverdi (OEL STEL) | 175 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) [ppm] | 37.5 ppm (value calculated) |
| OEL chemical category | skin notation |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA [ppm] | 20 ppm (Turpentine and selected Monoterpenes) |
| ACGIH chemical category | Not Classifiable as a Human Carcinogen, dermal sensitizer |

| .beta.-Pinene (127-91-3) |  |
| --- | --- |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA [ppm] | 20 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 150 mg/m³ (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| OEL TWA [ppm] | 25 ppm (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| OEL STEL | 300 mg/m³ (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| OEL STEL [ppm] | 50 ppm (Turpentine produced from Nordic conifers has an irritating effect on the skin, monoterpenes, with the exception of 3-Carene, have a lesser effect) |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 150 mg/m³ |
| IPRV (OEL TWA) [ppm] | 25 ppm |
| TPRV (OEL STEL) | 300 mg/m³ |
| TPRV (OEL STEL) [ppm] | 50 ppm |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA [ppm] | 20 ppm (Turpentine and selected Monoterpenes) |
| OEL chemical category | Sensitizer dermal, A4 - Not Classifiable as a Human Carcinogen |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) [1] | 113 mg/m³ |
| VLA-ED (OEL TWA) [2] | 20 ppm |
| OEL chemical category | Sensitizer |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 150 mg/m³ |
| NGV (OEL TWA) [ppm] | 25 ppm |
| KTV (OEL STEL) | 300 mg/m³ |
| KTV (OEL STEL) [ppm] | 50 ppm |
| OEL chemical category | Sensitizer |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) [1] | 140 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 25 ppm |
| Korttidsverdi (OEL STEL) | 175 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) [ppm] | 37.5 ppm (value calculated) |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA [ppm] | 20 ppm (Turpentine and selected Monoterpenes) |
| ACGIH chemical category | Not Classifiable as a Human Carcinogen, dermal sensitizer |

| decyl alcohol (112-30-1) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) [1] | 66 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) [2] | 10 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 10 mg/m³ |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 100 mg/m³ |
| OEL TWA [ppm] | 15 ppm |
| OEL STEL | 200 mg/m³ |
| OEL STEL [ppm] | 30 ppm |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) [1] | 66 mg/m³ (aerosol, vapour) |
| MAK (OEL TWA) [2] | 10 ppm (aerosol, vapour) |
| KZGW (OEL STEL) | 66 mg/m³ (aerosol, vapour) |
| KZGW (OEL STEL) [ppm] | 10 ppm (aerosol, vapour) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL STEL) | 42 mg/m³ |
| HTP (OEL STEL) [ppm] | 10 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 40 mg/m³ |
| NDSCh (OEL STEL) | 80 mg/m³ |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 5 mg/m³ |

| CAFFEINE (58-08-2) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 0.5 mg/m³ |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 0.5 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 0.5 mg/m³ (base) |

| Butyric acid (107-92-6) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 10 mg/m³ |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 15 mg/m³ |
| OEL TWA [ppm] | 4 ppm |
| OEL STEL | 30 mg/m³ |
| OEL STEL [ppm] | 8 ppm |

8.1.2. Recommended monitoring procedures

No additional information available

8.1.3. Air contaminants formed

No additional information available

8.1.4. DNEL and PNEC

No additional information available

8.1.5. Control banding

No additional information available

8.2. Exposure controls

8.2.1. Appropriate engineering controls

No additional information available

8.2.2. Personal protection equipment

Personal protective equipment:

Avoid all unnecessary exposure.

Personal protective equipment symbol(s):

8.2.2.1. Eye and face protection

Eye protection:

Chemical goggles or safety glasses

8.2.2.2. Skin protection

Hand protection:

Wear protective gloves.

8.2.2.3. Respiratory protection

Respiratory protection:

Wear appropriate mask

8.2.2.4. Thermal hazards

No additional information available

8.2.3. Environmental exposure controls

Other information:

Do not eat, drink or smoke during use.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

| Physical state | : | Liquid |
| --- | --- | --- |
| Color | : | Standard. |
| Odor | : | characteristic. |
| Odor threshold | : | No data available |
| pH | : | No data available |
| Relative evaporation rate (butyl acetate=1) | : | No data available |
| Melting point | : | No data available |
| Freezing point | : | No data available |
| Boiling point | : | No data available |
| Flash point | : | > 93 °C |
| Auto-ignition temperature | : | No data available |
| Decomposition temperature | : | No data available |
| Flammability (solid, gas) | : | Non flammable. |
| Vapor pressure | : | No data available |
| Relative vapor density at 20°C | : | No data available |
| Relative density | : | No data available |
| Solubility | : | No data available |
| Partition coefficient n-octanol/water (Log Pow) | : | No data available |
| Viscosity, kinematic | : | No data available |
| Viscosity, dynamic | : | No data available |
| Explosive properties | : | No data available |
| Oxidizing properties | : | No data available |
| Explosion limits | : | No data available |

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

No additional information available

10.2. Chemical stability

Not established.

10.3. Possibility of hazardous reactions

Not established.

10.4. Conditions to avoid

Direct sunlight. Extremely high or low temperatures.

10.5. Incompatible materials

Strong acids. Strong bases.

10.6. Hazardous decomposition products

fume. Carbon monoxide. Carbon dioxide.

SECTION 11: Toxicological information

11.1 Information on toxicological effects

| Acute toxicity (oral) | : | Not classified |
| --- | --- | --- |
| Acute toxicity (dermal) | : | Not classified |
| Acute toxicity (inhalation) | : | Not classified |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| LD50 oral rat | 2490 mg/kg (Source: JAPAN_GHS) |
| LD50 oral | 2490 mg/kg body weight |
| LD50 dermal rabbit | > 5000 mg/kg (Source: JAPAN_GHS) |

| 2-Isobutyl-4-methyltetrahydro-2H-pyran-4-ol (63500-71-0) |  |
| --- | --- |
| LD50 dermal rabbit | > 2000 mg/kg (Source: ECHA_API) |

| Hexyl salicylate (6259-76-3) |  |
| --- | --- |
| LD50 oral rat | > 5 g/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 5000 mg/kg (Source: ECHA_API) |

| Hexyl cinnamic aldehyde (101-86-0) |  |
| --- | --- |
| LD50 oral rat | 3100 mg/kg (Source: NLM_CIP) |
| LD50 oral | 3100 mg/kg body weight |
| LD50 dermal rabbit | > 3000 mg/kg (Source: EPA_HPV) |
| LC50 Inhalation - Rat | > 5 mg/l/4h |

| Linalool (78-70-6) |  |
| --- | --- |
| LD50 oral | 2790 mg/kg body weight |

| Hexamethylindanopyran (1222-05-5) |  |
| --- | --- |
| LD50 oral rat | > 3250 mg/kg (Source: CHEMVIEW) |
| LD50 dermal rabbit | > 3250 mg/kg (Source: CHEMVIEW) |

| Linalyl acetate (115-95-7) |  |
| --- | --- |
| LD50 oral rat | 14550 mg/kg (Source: EPA_HPV) |
| LD50 dermal rabbit | > 5000 mg/kg (Source: EPA_HPV) |

| d-Limonene (5989-27-5) |  |
| --- | --- |
| LD50 oral rat | 4400 mg/kg (Source: CHEMVIEW) |
| LD50 dermal rabbit | > 5 g/kg (Source: CHEMVIEW) |

| Citral (5392-40-5) |  |
| --- | --- |
| LD50 oral rat | 4960 mg/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | 2250 mg/kg (Source: NLM_CIP) |

| Orange oil (8008-57-9) |  |
| --- | --- |
| LD50 oral rat | 4400 mg/kg (Source: NZ_CCID) |
| LD50 dermal rabbit | > 5000 mg/kg (Source: CHEMVIEW) |

| Helional (1205-17-0) |  |
| --- | --- |
| LD50 dermal rabbit | > 2000 mg/kg (Source: ECHA_API) |

| Geraniol (106-24-1) |  |
| --- | --- |
| LD50 oral rat | 3600 mg/kg (Source: NLM_CIP) |
| LD50 oral | 3600 mg/kg body weight |
| LD50 dermal rabbit | > 5 g/kg (Source: NLM_CIP) |

| Aldehyde C-16 (77-83-8) |  |
| --- | --- |
| LD50 oral rat | 5470 mg/kg (Source: NLM_CIP) |
| LD50 dermal rat | > 2000 mg/kg (Source: ECHA_API) |

| Benzyl benzoate (120-51-4) |  |
| --- | --- |
| LD50 oral rat | 500 mg/kg (Source: NLM_CIP) |
| LD50 oral | 1160 mg/kg body weight |
| LD50 dermal rabbit | 4000 mg/kg (Source: NLM_CIP) |

| Triplal (Vertocitral) (68039-49-6) |  |
| --- | --- |
| LD50 oral | 3900 mg/kg body weight |

| Patchouli oil (8014-09-3) |  |
| --- | --- |
| LD50 oral rat | > 5 g/kg (Source: NLM_CIP) |

| Allyl caproate (123-68-2) |  |
| --- | --- |
| LD50 oral | 300 mg/kg body weight |
| LD50 dermal rabbit | 820 mg/kg (Source: ECHA_API) |
| LD50 dermal | 300 mg/kg body weight |
| LC50 Inhalation - Rat (Vapours) | 3 mg/l/4h |

| Ethyl alcohol (64-17-5) |  |
| --- | --- |
| LD50 oral rat | 7060 mg/kg (Source: NLM_CIP) |
| LC50 Inhalation - Rat | 133.8 mg/l/4h |

| 1,2-Propanediol (57-55-6) |  |
| --- | --- |
| LD50 oral rat | 20 g/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | 20800 mg/kg (Source: NLM_CIP) |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| LD50 oral rat | 5.35 g/kg (Source: NLM_HSDB) |
| LD50 dermal rabbit | 9500 mg/kg (Source: NLM_CIP) |

| .alpha.-Pinene (80-56-8) |  |
| --- | --- |
| LD50 oral rat | 3700 mg/kg (Source: NLM_CIP) |
| LD50 oral | 500 mg/kg body weight |
| LD50 dermal rat | > 5000 mg/kg (Source: CHEMVIEW) |

| .beta.-Pinene (127-91-3) |  |
| --- | --- |
| LD50 oral rat | > 5000 mg/kg (Source: EPA_HPV) |
| LD50 dermal rabbit | > 5000 mg/kg (Source: CHEMVIEW) |

| decyl alcohol (112-30-1) |  |
| --- | --- |
| LD50 oral rat | 4720 mg/kg (Source: NZ_CCID) |
| LD50 dermal rabbit | 3560 mg/kg (Source: NLM_CIP) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| LD50 oral rat | 4890 mg/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 8100 mg/kg (Source: ECHA_API) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| LD50 oral rat | 3 g/kg (Source: NLM_HSDB) |
| LD50 oral | 4000 mg/kg body weight |
| LD50 dermal rabbit | 630 mg/kg (Source: NLM_HSDB) |

| CAFFEINE (58-08-2) |  |
| --- | --- |
| LD50 oral rat | 367.7 mg/kg (Source: OECD_SIDS) |
| LD50 oral | 370 mg/kg body weight |
| LD50 dermal rat | > 2000 mg/kg (Source: ECHA_API) |
| LC50 Inhalation - Rat | 4.94 mg/l/4h |

| Butyric acid (107-92-6) |  |
| --- | --- |
| LD50 oral rat | 2 g/kg (Source: NLM_CIP) |
| LD50 oral | 1630 mg/kg body weight |
| LD50 dermal rabbit | 530 mg/kg (Source: NLM_HSDB) |

| Skin corrosion/irritation | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Serious eye damage/irritation | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Respiratory or skin sensitization | : | May cause an allergic skin reaction. |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Germ cell mutagenicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Carcinogenicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| d-Limonene (5989-27-5) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| CAFFEINE (58-08-2) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| Reproductive toxicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| STOT-single exposure | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| STOT-repeated exposure | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Aspiration hazard | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Orange oil (8008-57-9) |  |
| --- | --- |
| Hydrocarbon | Yes |

| Benzyl benzoate (120-51-4) |  |
| --- | --- |
| Viscosity, kinematic | 7.456 mm²/s |

| Potential Adverse human health effects and symptoms | : | Based on available data, the classification criteria are not met |
| --- | --- | --- |

SECTION 12: Ecological information

12.1. Toxicity

| Hazardous to the aquatic environment, short–term (acute) | : | Not classified |
| --- | --- | --- |
| Hazardous to the aquatic environment, long–term (chronic) | : | Toxic to aquatic life with long lasting effects. |

| Linalool (78-70-6) |  |
| --- | --- |
| EC50 96h - Algae [1] | 88.3 mg/l (Species: Desmodesmus subspicatus) |

| Hexamethylindanopyran (1222-05-5) |  |
| --- | --- |
| LC50 - Fish [1] | 0.452 mg/l Wolf, 1996d-27682 |
| LC50 - Other aquatic organisms [1] | > 0.14 mg/l REACH DOSSIER Pimephales promelas |
| EC50 - Crustacea [2] | 260 μg/l REACH Dossier |
| EC50 - Other aquatic organisms [1] | 0.131 mg/l REACH Dossier |

| Linalyl acetate (115-95-7) |  |
| --- | --- |
| LC50 - Fish [1] | 11 mg/l (Exposure time: 96 h - Species: Cyprinus carpio [flow-through] Source: ECHA) |

| d-Limonene (5989-27-5) |  |
| --- | --- |
| LC50 - Fish [1] | 0.619 – 0.796 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 35 mg/l (Exposure time: 96 h - Species: Oncorhynchus mykiss Source: EPA) |

| Citral (5392-40-5) |  |
| --- | --- |
| EC50 - Crustacea [1] | 7 mg/l (Exposure time: 48 h - Species: Daphnia magna) |
| EC50 72h - Algae [1] | 16 mg/l (Species: Desmodesmus subspicatus) |
| EC50 96h - Algae [1] | 19 mg/l (Species: Desmodesmus subspicatus) |

| Geraniol (106-24-1) |  |
| --- | --- |
| LC50 - Fish [1] | 22 mg/l (Exposure time: 96 h - Species: Danio rerio [static] Source: ECHA) |

| Aldehyde C-16 (77-83-8) |  |
| --- | --- |
| LC50 - Fish [1] | 4.2 mg/l (Exposure time: 96 h - Species: Oncorhynchus mykiss [semi-static] Source: ECHA) |

| Benzyl benzoate (120-51-4) |  |
| --- | --- |
| LC50 - Fish [1] | 2.32 mg/l (Exposure time: 96 h - Species: Danio rerio [semi-static] Source: ECHA) |
| NOEC (chronic) | 0.168 mg/l |

| Allyl caproate (123-68-2) |  |
| --- | --- |
| LC50 - Fish [1] | 0.117 mg/l (Exposure time: 96 h - Species: Danio rerio [semi-static] Source: ECHA) |

| Ethyl alcohol (64-17-5) |  |
| --- | --- |
| LC50 - Fish [2] | > 100 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static] Source: EPA) |
| EC50 - Crustacea [1] | 9268 – 14221 mg/l (Exposure time: 48 h - Species: Daphnia magna) |
| EC50 - Crustacea [2] | 2 mg/l (Exposure time: 48 h - Species: Daphnia magna [Static]) |

| 1,2-Propanediol (57-55-6) |  |
| --- | --- |
| LC50 - Fish [1] | 51600 mg/l (Exposure time: 96 h - Species: Oncorhynchus mykiss [static] Source: IUCLID) |
| LC50 - Fish [2] | 41 – 47 ml/l (Exposure time: 96 h - Species: Oncorhynchus mykiss [static] Source: EPA) |
| EC50 - Crustacea [1] | > 1000 mg/l (Exposure time: 48 h - Species: Daphnia magna [Static]) |
| EC50 96h - Algae [1] | 19000 mg/l (Species: Pseudokirchneriella subcapitata) |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| LC50 - Fish [1] | > 10000 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static]) |
| EC50 - Crustacea [1] | 1919 mg/l (Exposure time: 48 h - Species: Daphnia magna) |

| .alpha.-Pinene (80-56-8) |  |
| --- | --- |
| LC50 - Fish [1] | 0.28 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static] Source: IUCLID) |
| EC50 - Crustacea [1] | 41 mg/l (Exposure time: 48 h - Species: Daphnia magna) |

| decyl alcohol (112-30-1) |  |
| --- | --- |
| LC50 - Fish [1] | 2.2 – 2.5 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 4.12 – 6.2 mg/l (Exposure time: 96 h - Species: Lepomis macrochirus [static] Source: EPA) |
| EC50 - Crustacea [1] | 3 mg/l (Exposure time: 48 h - Species: Daphnia magna) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| LC50 - Fish [1] | 12 – 16.5 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| LC50 - Fish [1] | 306 – 334 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 88 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static] Source: EPA) |

| CAFFEINE (58-08-2) |  |
| --- | --- |
| LC50 - Fish [1] | 151 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through]) |

| Butyric acid (107-92-6) |  |
| --- | --- |
| EC50 72h - Algae [1] | 46.7 mg/l (Species: Desmodesmus subspicatus) |

12.2. Persistence and degradability

| ACAI BERRY BOOST #EU55090F 25% |  |
| --- | --- |
| Persistence and degradability | Not established. |

| Benzyl benzoate (120-51-4) |  |
| --- | --- |
| Persistence and degradability | May cause long-term adverse effects in the environment. |

12.3. Bioaccumulative potential

| ACAI BERRY BOOST #EU55090F 25% |  |
| --- | --- |
| Bioaccumulative potential | Not established. |

| Benzyl acetate (140-11-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.96 (at 25 °C (at pH 7) |

| 2-Isobutyl-4-methyltetrahydro-2H-pyran-4-ol (63500-71-0) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.65 (at 23 °C (at pH >6.09-<6.74) |

| Hexyl salicylate (6259-76-3) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 5.5 (at 30 °C (at pH 7) |

| Hexamethylindanopyran (1222-05-5) |  |
| --- | --- |
| BCF - Fish [1] | (1618 dimensionless (whole body w.w.) |
| Partition coefficient n-octanol/water (Log Pow) | 5.3 (at 25 °C (at pH 7) |

| Linalyl acetate (115-95-7) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.9 (at 25 °C) |

| d-Limonene (5989-27-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.38 (at 37 °C (at pH 7.2) |

| Citral (5392-40-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.76 (at 25 °C) |

| Helional (1205-17-0) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.4 (at 25 °C) |

| Geraniol (106-24-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.6 (at 25 °C) |

| Aldehyde C-16 (77-83-8) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.4 (at 25 °C (cis isomer) |

| Benzyl benzoate (120-51-4) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.97 (at 25 °C) |
| Bioaccumulative potential | Not established. |

| Allyl caproate (123-68-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.191 (at 20 °C (at pH 5) |

| Ethyl alcohol (64-17-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | -0.35 (at 24 °C (at pH 7.4) |

| 1,2-Propanediol (57-55-6) |  |
| --- | --- |
| BCF - Fish [1] | (1 dimensionless) |
| Partition coefficient n-octanol/water (Log Pow) | -1.07 (at 20.5 °C (at pH >=6.2-<=6.4) |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 0.35 (at 25 °C (at pH 7) |

| .alpha.-Pinene (80-56-8) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.1 |

| decyl alcohol (112-30-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.5 (at 25 °C (at pH 6) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.3 (at 25 °C (at pH 5) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.88 |

| CAFFEINE (58-08-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | -0.091 (at 23 °C) |

| Butyric acid (107-92-6) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.1 (at 25 °C (at pH 3) |

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

No additional information available

12.6. Other adverse effects

| Additional information | : | Avoid release to the environment. |
| --- | --- | --- |

SECTION 13: Disposal considerations

13.1. Waste treatment methods

| Product/Packaging disposal recommendations | : | Dispose in a safe manner in accordance with local/national regulations. |
| --- | --- | --- |
| Ecology - waste materials | : | Avoid release to the environment. |
| HP code | : | HP3 - “Flammable:” – flammable liquid waste: liquid waste having a flash point below 60 °C or waste gas oil, diesel and light heating oils having a flash point > 55 °C and ≤ 75 °C; – flammable pyrophoric liquid and solid waste: solid or liquid waste which, even in small quantities, is liable to ignite within five minutes after coming into contact with air; – flammable solid waste: solid waste which is readily combustible or may cause or contribute to fire through friction; – flammable gaseous waste: gaseous waste which is flammable in air at 20 °C and a standard pressure of 101.3 kPa; – water reactive waste: waste which, in contact with water, emits flammable gases in dangerous quantities; – other flammable waste: flammable aerosols, flammable self-heating waste, flammable organic peroxides and flammable self-reactive waste. HP14 - “Ecotoxic:” waste which presents or may present immediate or delayed risks for one or more sectors of the environment |

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number |  |  |  |  |
| UN 3082 | UN 3082 | UN 3082 | UN 3082 | UN 3082 |
| 14.2. UN proper shipping name |  |  |  |  |
| ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super) | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super) | Environmentally hazardous substance, liquid, n.o.s. (Iso E Super) | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super) | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super) |
| Transport document description |  |  |  |  |
| UN 3082 ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super), 9, III, (-) | UN 3082 ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super), 9, III, MARINE POLLUTANT | UN 3082 Environmentally hazardous substance, liquid, n.o.s. (Iso E Super), 9, III | UN 3082 ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super), 9, III | UN 3082 ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (Iso E Super), 9, III |
| 14.3. Transport hazard class(es) |  |  |  |  |
| 9 | 9 | 9 | 9 | 9 |
| 14.4. Packing group |  |  |  |  |
| III | III | III | III | III |
| 14.5. Environmental hazards |  |  |  |  |
| Dangerous for the environment: Yes | Dangerous for the environment: Yes Marine pollutant: Yes | Dangerous for the environment: Yes | Dangerous for the environment: Yes | Dangerous for the environment: Yes |
| No supplementary information available |  |  |  |  |

14.6. Special precautions for user

| Overland transport |  |  |
| --- | --- | --- |
| Classification code (ADR) | : | M6 |
| Special provision (ADR) | : | 274, 335, 375, 601 |
| Limited quantities (ADR) | : | 5l |
| Excepted quantities (ADR) | : | E1 |
| Packing instructions (ADR) | : | P001, IBC03, LP01, R001 |
| Special packing provisions (ADR) | : | PP1 |
| Mixed packing provisions (ADR) | : | MP19 |
| Portable tank and bulk container instructions (ADR) | : | T4 |
| Portable tank and bulk container special provisions (ADR) | : | TP1, TP29 |
| Tank code (ADR) | : | LGBV |
| Vehicle for tank carriage | : | AT |
| Transport category (ADR) | : | 3 |
| Special provisions for carriage - Packages (ADR) | : | V12 |
| Special provisions for carriage - Loading, unloading and handling (ADR) | : | CV13 |
| Hazard identification number (Kemler No.) | : | 90 |
| Orange plates | : |  |
| Tunnel restriction code (ADR) | : | - |
| EAC | : | •3Z |

| Transport by sea |  |  |
| --- | --- | --- |
| Special provision (IMDG) | : | 274, 335, 969 |
| Limited quantities (IMDG) | : | 5 L |
| Excepted quantities (IMDG) | : | E1 |
| Packing instructions (IMDG) | : | LP01, P001 |
| Packing provisions (IMDG) | : | PP1 |
| IBC packing instructions (IMDG) | : | IBC03 |
| Tank instructions (IMDG) | : | T4 |
| Tank special provisions (IMDG) | : | TP1, TP29 |
| EmS-No. (Fire) | : | F-A |
| EmS-No. (Spillage) | : | S-F |
| Stowage category (IMDG) | : | A |

| Air transport |  |  |
| --- | --- | --- |
| PCA Excepted quantities (IATA) | : | E1 |
| PCA Limited quantities (IATA) | : | Y964 |
| PCA limited quantity max net quantity (IATA) | : | 30kgG |
| PCA packing instructions (IATA) | : | 964 |
| PCA max net quantity (IATA) | : | 450L |
| CAO packing instructions (IATA) | : | 964 |
| CAO max net quantity (IATA) | : | 450L |
| Special provision (IATA) | : | A97, A158, A197, A215 |
| ERG code (IATA) | : | 9L |

| Inland waterway transport |  |  |
| --- | --- | --- |
| Classification code (ADN) | : | M6 |
| Special provision (ADN) | : | 274, 335, 375, 601 |
| Limited quantities (ADN) | : | 5 L |
| Excepted quantities (ADN) | : | E1 |
| Carriage permitted (ADN) | : | T |
| Equipment required (ADN) | : | PP |
| Number of blue cones/lights (ADN) | : | 0 |

| Rail transport |  |  |
| --- | --- | --- |
| Classification code (RID) | : | M6 |
| Special provision (RID) | : | 274, 335, 375, 601 |
| Limited quantities (RID) | : | 5L |
| Excepted quantities (RID) | : | E1 |
| Packing instructions (RID) | : | P001, IBC03, LP01, R001 |
| Special packing provisions (RID) | : | PP1 |
| Mixed packing provisions (RID) | : | MP19 |
| Portable tank and bulk container instructions (RID) | : | T4 |
| Portable tank and bulk container special provisions (RID) | : | TP1, TP29 |
| Tank codes for RID tanks (RID) | : | LGBV |
| Transport category (RID) | : | 3 |
| Special provisions for carriage – Packages (RID) | : | W12 |
| Special provisions for carriage - Loading, unloading and handling (RID) | : | CW13, CW31 |
| Colis express (express parcels) (RID) | : | CE8 |
| Hazard identification number (RID) | : | 90 |

14.7. Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

15.1.1. EU-Regulations

REACH Annex XVII (Restriction List)

| EU restriction list (REACH Annex XVII) |  |  |
| --- | --- | --- |
| Reference code | Applicable on | Entry title or description |
| 3(a) | d-Limonene ; Orange oil ; Ethyl alcohol ; .alpha.-Pinene ; .beta.-Pinene ; Aldehyde C-6 | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 2.1 to 2.4, 2.6 and 2.7, 2.8 types A and B, 2.9, 2.10, 2.12, 2.13 categories 1 and 2, 2.14 categories 1 and 2, 2.15 types A to F |
| 3(b) | ACAI BERRY BOOST #EU55090F 25% ; 2-Isobutyl-4-methyltetrahydro-2H-pyran-4-ol ; Hexyl salicylate ; Iso E Super ; Hexyl cinnamic aldehyde ; Linalool ; CUPRESSUS FUNEBRIS WOOD OIL ; Linalyl acetate ; d-Limonene ; Citral ; Orange oil ; Helional ; Geraniol ; Aldehyde C-16 ; Benzyl benzoate ; Triplal (Vertocitral) ; Patchouli oil ; Allyl caproate ; Caproic acid ; CAFFEINE ; Butyric acid | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 3.1 to 3.6, 3.7 adverse effects on sexual function and fertility or on development, 3.8 effects other than narcotic effects, 3.9 and 3.10 |
| 3(c) | ACAI BERRY BOOST #EU55090F 25% ; Benzyl acetate ; Hexyl salicylate ; Iso E Super ; Hexyl cinnamic aldehyde ; CUPRESSUS FUNEBRIS WOOD OIL ; Hexamethylindanopyran ; d-Limonene ; Orange oil ; Helional ; Aldehyde C-16 ; Benzyl benzoate ; Triplal (Vertocitral) ; Patchouli oil ; Allyl caproate ; decyl alcohol | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard class 4.1 |
| 40. | d-Limonene ; Orange oil ; Ethyl alcohol ; .alpha.-Pinene ; .beta.-Pinene ; Aldehyde C-6 | Substances classified as flammable gases category 1 or 2, flammable liquids categories 1, 2 or 3, flammable solids category 1 or 2, substances and mixtures which, in contact with water, emit flammable gases, category 1, 2 or 3, pyrophoric liquids category 1 or pyrophoric solids category 1, regardless of whether they appear in Part 3 of Annex VI to Regulation (EC) No 1272/2008 or not. |

REACH Annex XIV (Authorisation List)

Contains no REACH Annex XIV substances.

REACH Candidate List (SVHC)

Contains no REACH candidate substance

PIC Regulation (Prior Informed Consent)

Contains no substance subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 July 2012 concerning the export and import of hazardous chemicals.

POP Regulation (Persistent Organic Pollutants)

Contains no substance(s) subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

Ozone Regulation (1005/2009)

Contains no substance subject to REGULATION (EU) No 1005/2009 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 September 2009 on substances that deplete the ozone layer.

Explosives Precursors Regulation (2019/1148)

Contains no substance subject to Regulation (EU) 2019/1148 of the European Parliament and of the Council of 20 June 2019 on the marketing and use of explosives precursors.

Drug Precursors Regulation (273/2004)

Contains no substance subject to Regulation (EC) 273/2004 of the European Parliament and of the Council of 11 February 2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances.

15.1.2. National regulations

| France |  |  |
| --- | --- | --- |
| Professional diseases. Code; Description. RG 84; Conditions caused by liquid organic solvents for professional use: saturated or unsaturated aliphatic or cyclic liquid hydrocarbons and mixtures thereof; liquid halogenated hydrocarbons; nitrated derivatives of aliphatic hydrocarbons; alcohols; glycols, glycol ethers; ketones; aldehydes; aliphatic and cyclic ethers, including tetrahydrofuran; esters; dimethylformamide and dimethylacetamine; acetonitrile and propionitrile; pyridine; dimethylsulfone and dimethylsulfoxide. |
| Germany |  |  |
| Water hazard class (WGK) | : | WGK 2, significant hazardous to water (Classification according to AwSV, Annex 1). |
| Storage class (LGK, TRGS 510) | : | LGK 12 - Non-combustible liquids. |
| Joint storage table | : |  |
| Joint storage not permitted for | : | LGK 1, LGK 6.2, LGK 7. |
| Joint storage with restrictions permitted for | : | LGK 4.1A, LGK 4.3, LGK 5.1C. |
| Joint storage permitted for | : | LGK 2A, LGK 2B, LGK 3, LGK 4.1B, LGK 4.2, LGK 5.1A, LGK 5.1B, LGK 5.2, LGK 6.1A, LGK 6.1B, LGK 6.1C, LGK 6.1D, LGK 8A, LGK 8B, LGK 10, LGK 11, LGK 12, LGK 13, LGK 10-13. |
| Hazardous Incident Ordinance (12. BImSchV) | : | Is not subject of the Hazardous Incident Ordinance (12. BImSchV) |
| Netherlands |  |  |
| ABM category | : | A(2) - toxic for aquatic organisms, may have longterm hazardous effects in aquatic environment |
| SZW-lijst van kankerverwekkende stoffen | : | CUPRESSUS FUNEBRIS WOOD OIL,Orange oil ,Triplal (Vertocitral),Ethyl alcohol are listed |
| SZW-lijst van mutagene stoffen | : | CUPRESSUS FUNEBRIS WOOD OIL,Orange oil ,Triplal (Vertocitral) are listed |
| SZW-lijst van reprotoxische stoffen – Borstvoeding | : | Ethyl alcohol is listed |
| SZW-lijst van reprotoxische stoffen – Vruchtbaarheid | : | Ethyl alcohol is listed |
| SZW-lijst van reprotoxische stoffen – Ontwikkeling | : | Ethyl alcohol is listed |
| Denmark |  |  |
| Classification remarks | : | Emergency management guidelines for the storage of flammable liquids must be followed |
| Danish National Regulations | : | Young people below the age of 18 years are not allowed to use the product Pregnant/breastfeeding women working with the product must not be in direct contact with the product |
| Switzerland |  |  |
| Storage class (LK) | : | LK 10/12 - Liquids |

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

| Data sources | : | REGULATION (EC) No 1272/2008 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 December 2008 on classification, labeling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006. |
| --- | --- | --- |
| Other information | : | None. |

| Full text of H- and EUH-phrases: |  |
| --- | --- |
| Acute Tox. 3 (Dermal) | Acute toxicity (dermal), Category 3 |
| Acute Tox. 3 (Inhalation) | Acute toxicity (inhalation) Category 3 |
| Acute Tox. 3 (Oral) | Acute toxicity (oral) Category 3 |
| Acute Tox. 4 (Oral) | Acute toxicity (oral) Category 4 |
| Aquatic Acute 1 | Hazardous to the aquatic environment – Acute Hazard Category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment – Chronic Hazard Category 1 |
| Aquatic Chronic 2 | Hazardous to the aquatic environment – Chronic Hazard Category 2 |
| Aquatic Chronic 3 | Hazardous to the aquatic environment – Chronic Hazard Category 3 |
| Asp. Tox. 1 | Aspiration hazard Category 1 |
| Eye Dam. 1 | Serious eye damage/eye irritation Category 1 |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 2 | Flammable liquids Category 2 |
| Flam. Liq. 3 | Flammable liquids Category 3 |
| H225 | Highly flammable liquid and vapor. |
| H226 | Flammable liquid and vapor. |
| H301 | Toxic if swallowed. |
| H302 | Harmful if swallowed. |
| H304 | May be fatal if swallowed and enters airways. |
| H311 | Toxic in contact with skin. |
| H314 | Causes severe skin burns and eye damage. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H318 | Causes serious eye damage. |
| H319 | Causes serious eye irritation. |
| H331 | Toxic if inhaled. |
| H361 | Suspected of damaging fertility or the unborn child. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H411 | Toxic to aquatic life with long lasting effects. |
| H412 | Harmful to aquatic life with long lasting effects. |
| Repr. 2 | Reproductive toxicity Category 2 |
| Skin Corr. 1B | Skin corrosion/irritation, Category 1, Sub-Category 1B |
| Skin Corr. 1C | Skin corrosion/irritation, Category 1, Sub-Category 1C |
| Skin Irrit. 1 | Skin corrosion/irritation Category 1 |
| Skin Irrit. 2 | Skin corrosion/irritation Category 2 |
| Skin Sens. 1 | Skin sensitization, Category 1 |
| Skin Sens. 1B | Skin sensitization, Category 1B |

| The classification complies with | : | ATP 12 |
| --- | --- | --- |

Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.