SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

| Product form | : | Mixture |
| --- | --- | --- |
| Product name | : | PEAR #EU7696F 25% in DPG |
| Product code | : | EU7696F_25% |
| Type of product | : | Perfumes, fragrances |

1.2. Relevant identified uses of the substance or mixture and uses advised against

1.2.1. Relevant identified uses

| Industrial/Professional use spec | : | Industrial For professional use only |
| --- | --- | --- |
| Use of the substance/mixture | : | Perfumes, fragrances |
| Function or use category | : | Odour agents |

1.2.2. Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

No additional information available

1.4. Emergency telephone number

No additional information available

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

| Skin sensitisation, Category 1 | H317 |  |  |
| --- | --- | --- | --- |
| Hazardous to the aquatic environment – Chronic Hazard, Category 3 | H412 |  |  |

Full text of H- and EUH-statements: see section 16

Adverse physicochemical, human health and environmental effects

No additional information available

2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

| Hazard pictograms (CLP) | : |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GHS07 |  |  |  |  |  |
| Signal word (CLP) | : | Warning |  |  |  |  |  |
| Contains | : | Geranyl acetate; Eugenol; Cinnamic aldehyde; Neryl acetate; (R)-p-mentha-1,8-diene; d-limonene; Hexyl cinnamic aldehyde; Triplal (Vertocitral) |  |  |  |  |  |
| Hazard statements (CLP) | : | H317 - May cause an allergic skin reaction. H412 - Harmful to aquatic life with long lasting effects. |  |  |  |  |  |
| Precautionary statements (CLP) | : | P261 - Avoid breathing dust/fume/gas/mist/vapours/spray. P272 - Contaminated work clothing should not be allowed out of the workplace. P273 - Avoid release to the environment. P280 - Wear protective gloves/protective clothing/eye protection/face protection/hearing protection. P302+P352 - IF ON SKIN: Wash with plenty of water. P321 - Specific treatment (see supplemental first aid instruction on this label). |  |  |  |  |  |
| Extra phrases | : | For professional users only. |  |  |  |  |  |

2.3. Other hazards

Contains no PBT and/or vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII

The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or substance(s) are not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %

SECTION 3: Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Verdox | CAS-No.: 88-41-5 EC-No.: 201-828-7 REACH-no: 01-2119970713-33 | 0.925 – 1.825 | Aquatic Chronic 2, H411 |
| Geranyl acetate | CAS-No.: 105-87-3 EC-No.: 203-341-5 REACH-no: 01-2119973480-35 | 0.825 – 1.65 | Skin Irrit. 2, H315 Skin Sens. 1, H317 Aquatic Chronic 3, H412 |
| Eugenol | CAS-No.: 97-53-0 EC-No.: 202-589-1 REACH-no: 01-2119971802-33 | 0.7 – 1.375 | Acute Tox. 4 (Oral), H302 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| Cinnamic aldehyde | CAS-No.: 104-55-2 EC-No.: 203-213-9 EC Index-No.: 606-155-00-6 REACH-no: 01-2119935242-45 | 0.7 – 1.375 | Acute Tox. 4 (Dermal), H312 Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1A, H317 Aquatic Chronic 3, H412 |
| Neryl acetate | CAS-No.: 141-12-8 EC-No.: 205-459-2 | 0.375 – 0.725 | Skin Sens. 1B, H317 |
| (R)-p-mentha-1,8-diene; d-limonene substance with national workplace exposure limit(s) (DE, ES, FI, SI, NO, CH) | CAS-No.: 5989-27-5 EC-No.: 205-341-0 EC Index-No.: 601-096-00-2 REACH-no: 01-2119493353-35 | 0.225 – 0.45 | Flam. Liq. 3, H226 Skin Irrit. 2, H315 Skin Sens. 1B, H317 Asp. Tox. 1, H304 Aquatic Acute 1, H400 Aquatic Chronic 3, H412 |
| isobutyl acetate substance with national workplace exposure limit(s) (AT, BE, BG, CY, CZ, DE, DK, EE, ES, FI, FR, GB, GR, HR, HU, IE, IT, LT, LU, LV, MT, NL, PL, PT, RO, SE, SI, SK, NO, CH, TR) | CAS-No.: 110-19-0 EC-No.: 203-745-1 EC Index-No.: 607-026-00-7 | 0.2 – 0.375 | Flam. Liq. 2, H225 STOT SE 3, H336 |
| Hexyl cinnamic aldehyde | CAS-No.: 101-86-0 EC-No.: 202-983-3 REACH-no: 01-2119533092-50 | 0.175 – 0.35 | Skin Sens. 1, H317 Aquatic Chronic 2, H411 |
| Butylated hydroxytoluene (BHT) crystals substance with national workplace exposure limit(s) (AT, BE, BG, DE, DK, ES, FI, FR, GB, GR, HR, IE, PT, SI, CH) | CAS-No.: 128-37-0 EC-No.: 204-881-4 REACH-no: 01-2119480433-40 | 0.125 – 0.225 | Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| Triplal (Vertocitral) | CAS-No.: 68039-49-6 EC-No.: 268-264-1 | 0.075 – 0.133 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1, H317 Aquatic Chronic 3, H412 |
| isopentyl acetate substance with national workplace exposure limit(s) (AT, BE, BG, CY, DE, DK, EE, ES, FI, FR, GI, GR, HR, HU, IE, IT, LT, LU, LV, MT, NL, PL, PT, RO, SE, SI, SK, NO, CH, TR); substance with a Community workplace exposure limit | CAS-No.: 123-92-2 EC-No.: 204-662-3 EC Index-No.: 607-130-00-2 REACH-no: 01-2119548408-32 | 0.05 – 0.1 | Flam. Liq. 3, H226 |
| Diphenyl oxide substance with national workplace exposure limit(s) (AT, BE, BG, CY, CZ, DE, DK, EE, ES, FI, FR, GB, GI, GR, HR, HU, IE, IT, LT, LU, LV, MT, NL, PL, PT, RO, SE, SI, SK, NO, CH, TR); substance with a Community workplace exposure limit | CAS-No.: 101-84-8 EC-No.: 202-981-2 REACH-no: 01-2119472545-33 | 0.05 – 0.075 | Eye Irrit. 2, H319 Aquatic Acute 1, H400 Aquatic Chronic 1, H410 |
| Amyl formate substance with national workplace exposure limit(s) (LT, LV) | CAS-No.: 638-49-3 EC-No.: 211-340-6 EC Index-No.: 607-696-00-0 | 0.025 – 0.05 | Flam. Liq. 3, H226 Skin Irrit. 2, H315 Eye Irrit. 2, H319 STOT SE 3, H335 |
| Alcohol C-10 substance with national workplace exposure limit(s) (BG, DE, LT, LV, RO, CH) | CAS-No.: 112-30-1 EC-No.: 203-956-9 | 0 – 0.00285 | Aquatic Chronic 3, H412 |
| Aldehyde C-6 substance with national workplace exposure limit(s) (FI, PL) | CAS-No.: 66-25-1 EC-No.: 200-624-5 | 0 – 0.0007 | Flam. Liq. 3, H226 |
| Dipropylene glycol monomethyl ether substance with national workplace exposure limit(s) (AT, BE, BG, CY, CZ, DE, DK, EE, ES, FI, FR, GB, GI, GR, HR, HU, IE, IT, LT, LU, LV, MT, NL, PL, PT, RO, SE, SI, SK, NO, CH, TR); substance with a Community workplace exposure limit | CAS-No.: 34590-94-8 EC-No.: 252-104-2 | 0.0001905 – 0.0003175 | Not classified |
| Caproic acid substance with national workplace exposure limit(s) (BG, LT, LV) | CAS-No.: 142-62-1 EC-No.: 205-550-7 | 0 – 0.000025 | Eye Dam. 1, H318 Skin Corr. 1C, H314 |
| butyric acid substance with national workplace exposure limit(s) (BG, LT, LV, RO) | CAS-No.: 107-92-6 EC-No.: 203-532-3 EC Index-No.: 607-135-00-X | 0 – 0.000025 | Skin Corr. 1B, H314 |
| Toluene substance with national workplace exposure limit(s) (AT, BE, BG, CY, CZ, DE, DK, EE, ES, FI, FR, GB, GI, GR, HR, HU, IE, IT, LT, LU, LV, MT, NL, PL, PT, RO, SE, SI, SK, NO, CH, TR); substance with a Community workplace exposure limit | CAS-No.: 108-88-3 EC-No.: 203-625-9 EC Index-No.: 601-021-00-3 | ≤ 0.00000375 | Flam. Liq. 2, H225 Skin Irrit. 2, H315 Repr. 2, H361d STOT SE 3, H336 STOT RE 2, H373 Asp. Tox. 1, H304 |

Full text of H- and EUH-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

| First-aid measures general | : | Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label where possible). |
| --- | --- | --- |
| First-aid measures after inhalation | : | Allow affected person to breathe fresh air. Allow the victim to rest. |
| First-aid measures after skin contact | : | Remove affected clothing and wash all exposed skin area with mild soap and water, followed by warm water rinse. |
| First-aid measures after eye contact | : | Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persists. |
| First-aid measures after ingestion | : | Rinse mouth. Do NOT induce vomiting. Obtain emergency medical attention. |

4.2. Most important symptoms and effects, both acute and delayed

| Symptoms/effects | : | Not expected to present a significant hazard under anticipated conditions of normal use. |
| --- | --- | --- |

4.3. Indication of any immediate medical attention and special treatment needed

No additional information available

SECTION 5: Firefighting measures

5.1. Extinguishing media

| Suitable extinguishing media | : | Foam. Dry powder. Carbon dioxide. Water spray. Sand. |
| --- | --- | --- |
| Unsuitable extinguishing media | : | Do not use a heavy water stream. |

5.2. Special hazards arising from the substance or mixture

No additional information available

5.3. Advice for firefighters

| Firefighting instructions | : | Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. Prevent fire fighting water from entering the environment. |
| --- | --- | --- |
| Protection during firefighting | : | Do not enter fire area without proper protective equipment, including respiratory protection. |

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

6.1.1. For non-emergency personnel

| Emergency procedures | : | Evacuate unnecessary personnel. |
| --- | --- | --- |

6.1.2. For emergency responders

| Protective equipment | : | Equip cleanup crew with proper protection. |
| --- | --- | --- |
| Emergency procedures | : | Ventilate area. |

6.2. Environmental precautions

Prevent entry to sewers and public waters. Notify authorities if liquid enters sewers or public waters.

6.3. Methods and material for containment and cleaning up

| Methods for cleaning up | : | Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Collect spillage. Store away from other materials. |
| --- | --- | --- |

6.4. Reference to other sections

See Section 8. Exposure controls and personal protection.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

| Precautions for safe handling | : | Wash hands and other exposed areas with mild soap and water before eating, drinking or smoking and when leaving work. Provide good ventilation in process area to prevent formation of vapour. |
| --- | --- | --- |

7.2. Conditions for safe storage, including any incompatibilities

| Storage conditions | : | Keep only in the original container in a cool, well ventilated place away from : Keep container closed when not in use. |
| --- | --- | --- |
| Incompatible products | : | Strong bases. Strong acids. |
| Incompatible materials | : | Sources of ignition. Direct sunlight. |
| Germany |  |  |
| Storage class (LGK, TRGS 510) | : | LGK 12 - Non-combustible liquids |
| Joint storage table | : |  |
| Joint storage not permitted for | : | LGK 1, LGK 6.2, LGK 7 |
| Joint storage with restrictions permitted for | : | LGK 4.1A, LGK 4.3, LGK 5.1C |
| Joint storage permitted for | : | LGK 2A, LGK 2B, LGK 3, LGK 4.1B, LGK 4.2, LGK 5.1A, LGK 5.1B, LGK 5.2, LGK 6.1A, LGK 6.1B, LGK 6.1C, LGK 6.1D, LGK 8A, LGK 8B, LGK 10, LGK 11, LGK 12, LGK 13, LGK 10-13 |
| Switzerland |  |  |
| Storage class (LK) | : | LK 10/12 - Liquids |

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

8.1.1 National occupational exposure and biological limit values

| (R)-p-mentha-1,8-diene; d-limonene (5989-27-5) |  |
| --- | --- |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 140 mg/m³ |
| HTP (OEL TWA) | 25 ppm |
| HTP (OEL STEL) | 280 mg/m³ |
| HTP (OEL STEL) | 50 ppm |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 28 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) | 5 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Chemical category | Skin notation, Skin sensitization |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 28 mg/m³ |
| OEL TWA | 5 ppm |
| OEL STEL | 112 mg/m³ |
| OEL STEL | 20 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 168 mg/m³ |
| VLA-ED (OEL TWA) | 30 ppm |
| OEL chemical category | Sensitizer, skin - potential for cutaneous absorption |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 140 mg/m³ |
| Grenseverdi (OEL TWA) | 25 ppm |
| Korttidsverdi (OEL STEL) | 175 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) | 37.5 ppm (value calculated) |
| OEL chemical category | Allergenic substance |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 40 mg/m³ |
| MAK (OEL TWA) | 7 ppm |
| KZGW (OEL STEL) | 80 mg/m³ |
| KZGW (OEL STEL) | 14 ppm |
| OEL chemical category | Sensitizer |

| isobutyl acetate (110-19-0) |  |
| --- | --- |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 241 mg/m³ (Butyl acetates) |
| MAK (OEL TWA) | 50 ppm (Butyl acetates) |
| MAK (OEL STEL) | 480 mg/m³ (Butyl acetate) |
| MAK (OEL STEL) | 100 ppm (Butyl acetate) |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 238 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 712 mg/m³ |
| OEL STEL | 150 ppm |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 241 mg/m³ |
| GVI (OEL TWA) | 50 ppm |
| KGVI (OEL STEL) | 723 mg/m³ |
| KGVI (OEL STEL) | 150 ppm |
| Cyprus - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 241 mg/m³ |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ (Butyl acetate, all isomers) |
| OEL TWA | 50 ppm (Butyl acetate, all isomers) |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 240 mg/m³ (Butyl acetate) |
| HTP (OEL TWA) | 50 ppm (Butyl acetate) |
| HTP (OEL STEL) | 725 mg/m³ (Butyl acetate) |
| HTP (OEL STEL) | 150 ppm (Butyl acetate) |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 241 mg/m³ (restrictive limit) |
| VME (OEL TWA) | 50 ppm (restrictive limit) |
| VLE (OEL C/STEL) | 723 mg/m³ (restrictive limit) |
| VLE (OEL C/STEL) | 150 ppm (restrictive limit) |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 300 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) | 62 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 241 mg/m³ |
| CK (OEL STEL) | 723 mg/m³ |
| OEL chemical category | Sensitizer |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ (calculated) |
| OEL STEL | 150 ppm (calculated) |
| Italy - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 241 mg/m³ |
| IPRV (OEL TWA) | 50 ppm |
| TPRV (OEL STEL) | 723 mg/m³ |
| TPRV (OEL STEL) | 150 ppm |
| Luxembourg - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Malta - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 241 mg/m³ |
| TGG-8u (OEL TWA) | 50 ppm |
| TGG-15min (OEL STEL) | 723 mg/m³ |
| TGG-15min (OEL STEL) | 150 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 240 mg/m³ |
| NDSCh (OEL STEL) | 720 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ (indicative limit value) |
| OEL TWA | 50 ppm (indicative limit value) |
| OEL STEL | 723 mg/m³ (indicative limit value) |
| OEL STEL | 150 ppm (indicative limit value) |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) | 241 mg/m³ |
| NPHV (OEL TWA) | 50 ppm |
| NPHV (OEL C) | 723 mg/m³ |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 241 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 723 mg/m³ |
| OEL STEL | 150 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 241 mg/m³ |
| VLA-ED (OEL TWA) | 50 ppm |
| VLA-EC (OEL STEL) | 723 mg/m³ |
| VLA-EC (OEL STEL) | 150 ppm |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 241 mg/m³ (Butyl acetates) |
| NGV (OEL TWA) | 50 ppm (Butyl acetates) |
| KGV (OEL STEL) | 723 mg/m³ (Butyl acetates) |
| KGV (OEL STEL) | 150 ppm (Butyl acetates) |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) | 724 mg/m³ |
| WEL TWA (OEL TWA) | 150 ppm |
| WEL STEL (OEL STEL) | 903 mg/m³ |
| WEL STEL (OEL STEL) | 187 ppm |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 241 mg/m³ |
| Grenseverdi (OEL TWA) | 50 ppm |
| Korttidsverdi (OEL STEL) | 723 mg/m³ (value from the regulation) |
| Korttidsverdi (OEL STEL) | 150 ppm (value from the regulation) |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 240 mg/m³ |
| MAK (OEL TWA) | 50 ppm |
| KZGW (OEL STEL) | 720 mg/m³ |
| KZGW (OEL STEL) | 150 ppm |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 50 ppm (Butyl acetates, all isomers) |
| ACGIH OEL STEL | 150 ppm (Butyl acetates, all isomers) |

| Butylated hydroxytoluene (BHT) crystals (128-37-0) |  |
| --- | --- |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 10 mg/m³ |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 2 mg/m³ (aerosol and vapor) |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| OEL STEL | 50 mg/m³ |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 10 mg/m³ |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| OEL STEL | 20 mg/m³ |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 10 mg/m³ |
| HTP (OEL STEL) | 20 mg/m³ |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 10 mg/m³ |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 10 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed-inhalable fraction) |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 2 mg/m³ |
| OEL STEL | 6 mg/m³ (calculated) |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 2 mg/m³ (inhalable fraction; vapor) |
| OEL chemical category | A4 - Not Classifiable as a Human Carcinogen |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ (inhalable fraction) |
| OEL STEL | 40 mg/m³ (inhalable fraction) |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 10 mg/m³ |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) | 10 mg/m³ |
| WEL STEL (OEL STEL) | 30 mg/m³ (calculated) |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 10 mg/m³ (no elevated carcinogenic risk by keeping the MAK-value-aerosol, inhalable dust, vapour) |
| KZGW (OEL STEL) | 40 mg/m³ (no increased cancer risk by adhering to TWA values-aerosol, inhalable dust, vapour) |
| OEL chemical category | Category C1B carcinogen carcinogenic with threshold value |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 2 mg/m³ (inhalable fraction and vapor) |
| ACGIH chemical category | Not Classifiable as a Human Carcinogen |

| Toluene (108-88-3) |  |
| --- | --- |
| EU - Indicative Occupational Exposure Limit (IOEL) |  |
| IOEL TWA | 192 mg/m³ |
| IOEL TWA | 50 ppm |
| IOEL STEL | 384 mg/m³ |
| IOEL STEL | 100 ppm |
| Remark | Possibility of significant uptake through the skin |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 190 mg/m³ |
| MAK (OEL TWA) | 50 ppm |
| MAK (OEL STEL) | 380 mg/m³ |
| MAK (OEL STEL) | 100 ppm |
| OEL chemical category | Skin notation |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 77 mg/m³ |
| OEL TWA | 20 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Skin, Skin notation |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| Bulgaria - Biological limit values |  |
| BLV | 1.6 mmol/mmol Creatinine Parameter: Hippuric acid - Medium: urine - Sampling time: at the end of exposure or end of work shift |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 192 mg/m³ |
| GVI (OEL TWA) | 50 ppm |
| KGVI (OEL STEL) | 384 mg/m³ |
| KGVI (OEL STEL) | 100 ppm |
| OEL chemical category | Skin notation |
| Croatia - Biological limit values |  |
| BLV | 1 mg/l Parameter: Toluene - Medium: blood - Sampling time: at the end of the work shift 20 ppm Parameter: Toluene - Medium: final exhaled air - Sampling time: during exposure 2.5 g/g creatinine Parameter: Hippuric acid - Medium: urine - Sampling time: at the end of the work shift (calculated on the average Creatinine value of 1.2 g/L urine) 1 mg/g creatinine Parameter: o-Cresol - Medium: urine - Sampling time: at the end of the work shift (calculated on the average Creatinine value of 1.2 g/L urine) |
| Cyprus - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Skin-potential for cutaneous absorption |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 200 mg/m³ |
| OEL chemical category | Potential for cutaneous absorption |
| Czech Republic - Biological limit values |  |
| BLV | 1.6 µmol/mmol Creatinine Parameter: o-Cresol - Medium: urine - Sampling time: end of shift (after hydrolysis) 1000 µmol/mmol Creatinine Parameter: Hippuric acid - Medium: urine - Sampling time: end of shift (exposure testing using the o-Cresol parameter to precisely measure Toluene exposure is needed if the value of Hippuric acid is between 1600 and 2500 mg/g of Creatinine, no additional testing is needed if the Hippuric acid value is >2500 mg/g of Creatinine as work exposure to Toluene will have highly exceeded the PEL value.) 1.5 mg/g creatinine Parameter: o-Cresol - Medium: urine - Sampling time: end of shift (after hydrolysis) 1600 mg/g creatinine Parameter: Hippuric acid - Medium: urine - Sampling time: end of shift (exposure testing using the o-Cresol parameter to precisely measure Toluene exposure is needed if the value of Hippuric acid is between 1600 and 2500 mg/g of Creatinine, no additional testing is needed if the Hippuric acid value is >2500 mg/g of Creatinine as work exposure to Toluene will have highly exceeded the PEL value.) |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 94 mg/m³ |
| OEL TWA | 25 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Skin notation |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 81 mg/m³ |
| HTP (OEL TWA) | 25 ppm |
| HTP (OEL STEL) | 380 mg/m³ |
| HTP (OEL STEL) | 100 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Finland - Biological limit values |  |
| BLV | 500 nmol/L Parameter: Toluene - Medium: blood - Sampling time: in the morning after a working day |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 76.8 mg/m³ (restrictive limit) |
| VME (OEL TWA) | 20 ppm (restrictive limit) |
| VLE (OEL C/STEL) | 384 mg/m³ (restrictive limit) |
| VLE (OEL C/STEL) | 100 ppm (restrictive limit) |
| OEL chemical category | Reproductive Toxin category 2, Risk of cutaneous absorption |
| France - Biological limit values |  |
| BLV | 20 μg/l Parameter: Toluene - Medium: blood - Sampling time: end of workweek (Semi-quantitative (ambiguous interpretation)) Parameter: Hippuric acid - Medium: urine - Sampling time: end of shift (per the Authority, the values for this substance must be decided and/or determined on a case by case basis. Guidance for the calculation of and interpretation of values is provided in the source) |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 190 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) | 50 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Chemical category | Skin notation |
| Germany - Biological limit values (TRGS 903) |  |
| Biological limit value | 600 μg/l Parameter: Toluene - Medium: whole blood - Sampling time: immediately after exposure 75 μg/l Parameter: Toluene - Medium: urine - Sampling time: end of shift 1.5 mg/l Parameter: o-Cresol (after hydrolysis) - Medium: urine - Sampling time: for long-term exposures: at the end of the shift after several shifts 1.5 mg/l Parameter: o-Cresol (after hydrolysis) - Medium: urine - Sampling time: end of shift |
| Gibraltar - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Skin notation |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | skin - potential for cutaneous absorption |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 190 mg/m³ |
| CK (OEL STEL) | 384 mg/m³ |
| OEL chemical category | Potential for cutaneous absorption |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Italy - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | skin - potential for cutaneous absorption |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 50 mg/m³ |
| OEL TWA | 14 ppm |
| OEL chemical category | skin - potential for cutaneous exposure |
| Latvia - Biological Exposure Indices |  |
| BEI | 1.6 g/g creatinine Parameter: Hippuric acid - Medium: urine - Sampling time: end of shift 0.05 mg/l Parameter: Toluene - Medium: blood - Sampling time: end of shift |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 192 mg/m³ |
| IPRV (OEL TWA) | 50 ppm |
| TPRV (OEL STEL) | 384 mg/m³ |
| TPRV (OEL STEL) | 100 ppm |
| OEL chemical category | Reproductive toxin, Skin notation |
| Luxembourg - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Possibility of significant uptake through the skin |
| Malta - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Possibility of significant uptake through the skin |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 150 mg/m³ |
| TGG-8u (OEL TWA) | 39 ppm |
| TGG-15min (OEL STEL) | 384 mg/m³ |
| TGG-15min (OEL STEL) | 100 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 100 mg/m³ |
| NDSCh (OEL STEL) | 200 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ (indicative limit value) |
| OEL TWA | 50 ppm (indicative limit value) |
| OEL STEL | 384 mg/m³ (indicative limit value) |
| OEL STEL | 100 ppm (indicative limit value) |
| OEL chemical category | A4 - Not Classifiable as a Human Carcinogen, skin - potential for cutaneous exposure indicative limit value |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Skin notation |
| Romania - Biological limit values |  |
| BLV | 2 g/l Parameter: Hippuric acid - Medium: urine - Sampling time: end of shift 3 mg/l Parameter: o-Cresol - Medium: urine - Sampling time: end of shift |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) | 192 mg/m³ |
| NPHV (OEL TWA) | 50 ppm |
| NPHV (OEL C) | 384 mg/m³ (also biological monitoring considered) |
| OEL chemical category | Potential for cutaneous absorption |
| Slovakia - Biological limit values |  |
| BLV | 600 μg/l Parameter: Toluene - Medium: blood - Sampling time: end of exposure or work shift 1.5 mg/l Parameter: o-Cresol - Medium: urine - Sampling time: after all work shifts (for long-term exposure) 1.5 mg/l Parameter: o-Cresol - Medium: urine - Sampling time: end of exposure or work shift 2401 mg/g creatinine Parameter: Hippuric acid - Sampling time: end of exposure or work shift |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 192 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 384 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Category 2, Potential for cutaneous absorption |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 192 mg/m³ (indicative limit value) |
| VLA-ED (OEL TWA) | 50 ppm (indicative limit value) |
| VLA-EC (OEL STEL) | 384 mg/m³ |
| VLA-EC (OEL STEL) | 100 ppm |
| OEL chemical category | skin - potential for cutaneous absorption |
| Spain - Biological limit values |  |
| BLV | 0.6 mg/l Parameter: o-Cresol - Medium: urine - Sampling time: end of shift 0.05 mg/l Parameter: Toluene - Medium: blood - Sampling time: start of last shift of workweek 0.08 mg/l Parameter: Toluene - Medium: urine - Sampling time: end of shift |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 192 mg/m³ |
| NGV (OEL TWA) | 50 ppm |
| KGV (OEL STEL) | 384 mg/m³ |
| KGV (OEL STEL) | 100 ppm |
| OEL chemical category | Skin notation |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) | 191 mg/m³ |
| WEL TWA (OEL TWA) | 50 ppm |
| WEL STEL (OEL STEL) | 384 mg/m³ |
| WEL STEL (OEL STEL) | 100 ppm |
| WEL chemical category | Potential for cutaneous absorption |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 94 mg/m³ |
| Grenseverdi (OEL TWA) | 25 ppm |
| Korttidsverdi (OEL STEL) | 141 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) | 37.5 ppm (value calculated) |
| OEL chemical category | Skin notation |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 190 mg/m³ |
| MAK (OEL TWA) | 50 ppm |
| KZGW (OEL STEL) | 760 mg/m³ |
| KZGW (OEL STEL) | 200 ppm |
| OEL chemical category | Skin notation, Category 2 reproductive toxin |
| Switzerland - BAT |  |
| BAT | 600 μg/l Parameter: Toluene - Medium: whole blood - Sampling time: end of shift 6.48 µmol/l Parameter: Toluene - Medium: whole blood - Sampling time: end of shift 2 g/g creatinine Parameter: Hippuric acid - Medium: urine - Sampling time: end of shift, and after several shifts (for long-term exposures) Parameter: Hippuric acid - Medium: urine - Sampling time: end of shift, and after several shifts (for long-term exposures) 0.5 mg/l Parameter: o-Cresol - Medium: urine - Sampling time: end of shift, and after several shifts (for long-term exposures) 4.62 µmol/l Parameter: o-Cresol - Medium: urine - Sampling time: end of shift, and after several shifts (for long-term exposures) 75 μg/l Parameter: Toluol - Medium: urine - Sampling time: end of shift |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 20 ppm |
| ACGIH chemical category | Not Classifiable as a Human Carcinogen |
| USA - ACGIH - Biological Exposure Indices |  |
| BEI | 0.02 mg/l Parameter: Toluene - Medium: blood - Sampling time: prior to last shift of workweek 0.03 mg/l Parameter: Toluene - Medium: urine - Sampling time: end of shift 0.3 mg/g creatinine Parameter: o-Cresol with hydrolysis - Medium: urine - Sampling time: end of shift (background) |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| EU - Indicative Occupational Exposure Limit (IOEL) |  |
| IOEL TWA | 308 mg/m³ |
| IOEL TWA | 50 ppm |
| Remark | Possibility of significant uptake through the skin |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 307 mg/m³ (mixed isomers) |
| MAK (OEL TWA) | 50 ppm (mixed isomers) |
| MAK (OEL STEL) | 614 mg/m³ (isomers mixtures) |
| MAK (OEL STEL) | 100 ppm (isomers mixtures) |
| OEL chemical category | Skin notation |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | Skin, Skin notation |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 308 mg/m³ |
| GVI (OEL TWA) | 50 ppm |
| OEL chemical category | Skin notation |
| Cyprus - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | Skin-potential for cutaneous absorption |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 270 mg/m³ |
| OEL chemical category | Potential for cutaneous absorption |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 309 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 618 mg/m³ |
| OEL STEL | 100 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | Skin notation |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 310 mg/m³ |
| HTP (OEL TWA) | 50 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 308 mg/m³ (restrictive limit) |
| VME (OEL TWA) | 50 ppm (restrictive limit) |
| OEL chemical category | Risk of cutaneous absorption |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 310 mg/m³ (isomer mixture) |
| AGW (OEL TWA) | 50 ppm (isomer mixture) |
| Gibraltar - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | Skin notation |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 600 mg/m³ |
| OEL TWA | 100 ppm |
| OEL STEL | 900 mg/m³ |
| OEL STEL | 150 ppm |
| OEL chemical category | skin - potential for cutaneous absorption |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 308 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ ((2-Methoxymethylethoxy)propanol) |
| OEL TWA | 50 ppm ((2-Methoxymethylethoxy)propanol) |
| OEL STEL | 924 mg/m³ (calculated (2-(2-Methoxypropoxy)-1-propanol) |
| OEL STEL | 150 ppm (calculated (2-(2-Methoxypropoxy)-1-propanol) |
| OEL chemical category | Potential for cutaneous absorption |
| Italy - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ (1-(3-Methoxypropoxy)propan-1-ol) |
| OEL TWA | 50 ppm (1-(3-Methoxypropoxy)propan-1-ol) |
| OEL chemical category | skin - potential for cutaneous absorption |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | skin - potential for cutaneous exposure |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 300 mg/m³ (2-(2-Methoxypropoxy)-propanol) |
| IPRV (OEL TWA) | 50 ppm (2-(2-Methoxypropoxy)-propanol) |
| TPRV (OEL STEL) | 450 mg/m³ (2-(2-Methoxypropoxy)-propanol) |
| TPRV (OEL STEL) | 75 ppm (2-(2-Methoxypropoxy)-propanol) |
| OEL chemical category | Skin notation |
| Luxembourg - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | Possibility of significant uptake through the skin |
| Malta - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | Possibility of significant uptake through the skin |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 300 mg/m³ |
| TGG-8u (OEL TWA) | 48.7 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 240 mg/m³ (mixture of isomers: 1-(2-Methoxy-1-methylethoxy)propan-2-ol, 1-(2-Methoxy-2-methylethoxy)propan-2-ol and 2-(2-Methoxy-1-methylethoxy)propan-1-ol) |
| NDSCh (OEL STEL) | 480 mg/m³ (mixture of isomers: 1-(2-Methoxy-1-methylethoxy)propan-2-ol, 1-(2-Methoxy-2-methylethoxy)propan-2-ol, 2-(2-Methoxy-1-methylethoxy)propan-1-ol) |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ (indicative limit value) |
| OEL TWA | 50 ppm (indicative limit value) |
| OEL STEL | 150 ppm |
| OEL chemical category | skin - potential for cutaneous exposure indicative limit value |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL chemical category | Skin notation |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) | 308 mg/m³ |
| NPHV (OEL TWA) | 50 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 308 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 308 mg/m³ |
| OEL STEL | 50 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 308 mg/m³ (indicative limit value) |
| VLA-ED (OEL TWA) | 50 ppm (indicative limit value) |
| OEL chemical category | skin - potential for cutaneous absorption |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 300 mg/m³ |
| NGV (OEL TWA) | 50 ppm |
| KGV (OEL STEL) | 450 mg/m³ |
| KGV (OEL STEL) | 75 ppm |
| OEL chemical category | Skin notation |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) | 308 mg/m³ |
| WEL TWA (OEL TWA) | 50 ppm |
| WEL STEL (OEL STEL) | 924 mg/m³ (calculated) |
| WEL STEL (OEL STEL) | 150 ppm (calculated) |
| WEL chemical category | Potential for cutaneous absorption |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 300 mg/m³ |
| Grenseverdi (OEL TWA) | 50 ppm |
| Korttidsverdi (OEL STEL) | 375 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) | 75 ppm (value calculated) |
| OEL chemical category | Skin notation |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 300 mg/m³ (aerosol, vapour) |
| MAK (OEL TWA) | 50 ppm (aerosol, vapour) |
| KZGW (OEL STEL) | 300 mg/m³ (aerosol, vapour) |
| KZGW (OEL STEL) | 50 ppm (aerosol, vapour) |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 50 ppm (Dipropylene glycol methyl ether) |

| isopentyl acetate (123-92-2) |  |
| --- | --- |
| EU - Indicative Occupational Exposure Limit (IOEL) |  |
| IOEL TWA | 270 mg/m³ |
| IOEL TWA | 50 ppm |
| IOEL STEL | 540 mg/m³ |
| IOEL STEL | 100 ppm |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 270 mg/m³ (Pentyl acetate (all isomers)) |
| MAK (OEL TWA) | 50 ppm (Pentyl acetate (all isomers)) |
| MAK (OEL STEL) | 540 mg/m³ (Pentylacetate) |
| MAK (OEL STEL) | 100 ppm (Pentylacetate) |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 270 mg/m³ |
| GVI (OEL TWA) | 50 ppm |
| KGVI (OEL STEL) | 540 mg/m³ |
| KGVI (OEL STEL) | 100 ppm |
| Cyprus - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 271 mg/m³ (Amyl acetate, all isomers) |
| OEL TWA | 50 ppm (Amyl acetate, all isomers) |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 270 mg/m³ (Pentyl acetate) |
| HTP (OEL TWA) | 50 ppm (Pentyl acetate) |
| HTP (OEL STEL) | 540 mg/m³ |
| HTP (OEL STEL) | 100 ppm |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 270 mg/m³ (restrictive limit) |
| VME (OEL TWA) | 50 ppm (restrictive limit) |
| VLE (OEL C/STEL) | 540 mg/m³ (restrictive limit) |
| VLE (OEL C/STEL) | 100 ppm (restrictive limit) |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 270 mg/m³ |
| AGW (OEL TWA) | 50 ppm |
| Gibraltar - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 530 mg/m³ |
| OEL TWA | 100 ppm |
| OEL STEL | 800 mg/m³ |
| OEL STEL | 150 ppm |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 270 mg/m³ |
| CK (OEL STEL) | 540 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 260 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 520 mg/m³ |
| OEL STEL | 100 ppm |
| Italy - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 270 mg/m³ |
| IPRV (OEL TWA) | 50 ppm |
| TPRV (OEL STEL) | 540 mg/m³ |
| TPRV (OEL STEL) | 100 ppm |
| Luxembourg - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Malta - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Netherlands - Occupational Exposure Limits |  |
| TGG-15min (OEL STEL) | 530 mg/m³ |
| TGG-15min (OEL STEL) | 98.1 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 250 mg/m³ |
| NDSCh (OEL STEL) | 500 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ (indicative limit value) |
| OEL TWA | 50 ppm (indicative limit value (Pentyl acetate, all isomers) |
| OEL STEL | 540 mg/m³ (indicative limit value) |
| OEL STEL | 100 ppm (indicative limit value) |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) | 270 mg/m³ |
| NPHV (OEL TWA) | 50 ppm |
| NPHV (OEL C) | 540 mg/m³ |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 270 mg/m³ |
| OEL TWA | 50 ppm |
| OEL STEL | 540 mg/m³ |
| OEL STEL | 100 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 270 mg/m³ (indicative limit value) |
| VLA-ED (OEL TWA) | 50 ppm (indicative limit value) |
| VLA-EC (OEL STEL) | 540 mg/m³ |
| VLA-EC (OEL STEL) | 100 ppm |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 270 mg/m³ (Pentyl acetates) |
| NGV (OEL TWA) | 50 ppm (Pentyl acetates) |
| KGV (OEL STEL) | 540 mg/m³ (Pentyl acetates) |
| KGV (OEL STEL) | 100 ppm (Pentyl acetates) |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 260 mg/m³ |
| Grenseverdi (OEL TWA) | 50 ppm |
| Korttidsverdi (OEL STEL) | 325 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) | 75 ppm (value calculated) |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 260 mg/m³ (Pentyl acetate all isomers) |
| MAK (OEL TWA) | 50 ppm (Pentyl acetate all isomers) |
| KZGW (OEL STEL) | 260 mg/m³ (Pentyl acetate all isomers) |
| KZGW (OEL STEL) | 50 ppm (Pentyl acetate all isomers) |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 50 ppm (Pentyl acetate, all isomers) |
| ACGIH OEL STEL | 100 ppm (Pentyl acetate, all isomers) |

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| EU - Indicative Occupational Exposure Limit (IOEL) |  |
| IOEL TWA | 7 mg/m³ |
| IOEL TWA | 1 ppm |
| IOEL STEL | 14 mg/m³ |
| IOEL STEL | 2 ppm |
| Austria - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 7 mg/m³ |
| MAK (OEL TWA) | 1 ppm |
| MAK (OEL STEL) | 14 mg/m³ |
| MAK (OEL STEL) | 2 ppm |
| Belgium - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ (vapor) |
| OEL TWA | 1 ppm (vapor) |
| OEL STEL | 14 mg/m³ (vapor) |
| OEL STEL | 2 ppm (vapor) |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Croatia - Occupational Exposure Limits |  |
| GVI (OEL TWA) | 7 mg/m³ |
| GVI (OEL TWA) | 1 ppm |
| KGVI (OEL STEL) | 14 mg/m³ |
| KGVI (OEL STEL) | 2 ppm |
| Cyprus - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Czech Republic - Occupational Exposure Limits |  |
| PEL (OEL TWA) | 5 mg/m³ |
| Denmark - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Estonia - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) | 7 mg/m³ |
| HTP (OEL TWA) | 1 ppm |
| HTP (OEL STEL) | 14 mg/m³ |
| HTP (OEL STEL) | 2 ppm |
| France - Occupational Exposure Limits |  |
| VME (OEL TWA) | 7 mg/m³ (indicative limit) |
| VME (OEL TWA) | 1 ppm (indicative limit) |
| VLE (OEL C/STEL) | 14 mg/m³ (indicative limit) |
| VLE (OEL C/STEL) | 2 ppm (indicative limit) |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 7.1 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed-vapor) |
| AGW (OEL TWA) | 1 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed-vapor) |
| Gibraltar - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 200 ppm |
| Greece - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Hungary - Occupational Exposure Limits |  |
| AK (OEL TWA) | 7 mg/m³ |
| CK (OEL STEL) | 14 mg/m³ |
| Ireland - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ (vapour) |
| OEL TWA | 1 ppm (vapour) |
| OEL STEL | 14 mg/m³ (vapour) |
| OEL STEL | 2 ppm (vapour) |
| Italy - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 7 mg/m³ |
| IPRV (OEL TWA) | 1 ppm |
| TPRV (OEL STEL) | 14 mg/m³ |
| TPRV (OEL STEL) | 2 ppm |
| Luxembourg - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Malta - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Netherlands - Occupational Exposure Limits |  |
| TGG-8u (OEL TWA) | 7 mg/m³ |
| TGG-8u (OEL TWA) | 1 ppm |
| TGG-15min (OEL STEL) | 14 mg/m³ |
| TGG-15min (OEL STEL) | 2 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 7 mg/m³ |
| NDSCh (OEL STEL) | 14 mg/m³ |
| Portugal - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ (indicative limit value) |
| OEL TWA | 1 ppm (indicative limit value-vapor) |
| OEL STEL | 14 mg/m³ (indicative limit value) |
| OEL STEL | 2 ppm (indicative limit value-vapor) |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Slovakia - Occupational Exposure Limits |  |
| NPHV (OEL TWA) | 7 mg/m³ |
| NPHV (OEL TWA) | 1 ppm |
| NPHV (OEL C) | 14 mg/m³ |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 7 mg/m³ |
| OEL TWA | 1 ppm |
| OEL STEL | 14 mg/m³ |
| OEL STEL | 2 ppm |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) | 7.1 mg/m³ (vapor) |
| VLA-ED (OEL TWA) | 1 ppm (vapor) |
| VLA-EC (OEL STEL) | 14.2 mg/m³ (vapor) |
| VLA-EC (OEL STEL) | 2 ppm (vapor) |
| Sweden - Occupational Exposure Limits |  |
| NGV (OEL TWA) | 7 mg/m³ |
| NGV (OEL TWA) | 1 ppm |
| KGV (OEL STEL) | 14 mg/m³ |
| KGV (OEL STEL) | 2 ppm |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (OEL TWA) | 7 mg/m³ |
| WEL TWA (OEL TWA) | 1 ppm |
| WEL STEL (OEL STEL) | 14 mg/m³ |
| WEL STEL (OEL STEL) | 2 ppm |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) | 7 mg/m³ |
| Grenseverdi (OEL TWA) | 1 ppm |
| Korttidsverdi (OEL STEL) | 14 mg/m³ (value from the regulation) |
| Korttidsverdi (OEL STEL) | 2 ppm (value from the regulation) |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 7 mg/m³ (aerosol, vapour) |
| MAK (OEL TWA) | 1 ppm (aerosol, vapour) |
| KZGW (OEL STEL) | 14 mg/m³ (aerosol, vapour) |
| KZGW (OEL STEL) | 2 ppm (aerosol, vapour) |
| OEL chemical category | Category 2 reproductive toxin |
| USA - ACGIH - Occupational Exposure Limits |  |
| ACGIH OEL TWA | 1 ppm (vapor) |
| ACGIH OEL STEL | 2 ppm (vapor fraction) |

| Amyl formate (638-49-3) |  |
| --- | --- |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| TPRV (OEL STEL) | 10 mg/m³ |
| OEL chemical category | Skin notation |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) | 66 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) | 10 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 10 mg/m³ |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 100 mg/m³ |
| OEL TWA | 15 ppm |
| OEL STEL | 200 mg/m³ |
| OEL STEL | 30 ppm |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) | 66 mg/m³ (aerosol, vapour) |
| MAK (OEL TWA) | 10 ppm (aerosol, vapour) |
| KZGW (OEL STEL) | 66 mg/m³ (aerosol, vapour) |
| KZGW (OEL STEL) | 10 ppm (aerosol, vapour) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL STEL) | 42 mg/m³ |
| HTP (OEL STEL) | 10 ppm |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 40 mg/m³ |
| NDSCh (OEL STEL) | 80 mg/m³ |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 5 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 5 mg/m³ |

| butyric acid (107-92-6) |  |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Latvia - Occupational Exposure Limits |  |
| OEL TWA | 10 mg/m³ |
| Lithuania - Occupational Exposure Limits |  |
| IPRV (OEL TWA) | 10 mg/m³ |
| Romania - Occupational Exposure Limits |  |
| OEL TWA | 15 mg/m³ |
| OEL TWA | 4 ppm |
| OEL STEL | 30 mg/m³ |
| OEL STEL | 8 ppm |

8.1.2. Recommended monitoring procedures

No additional information available

8.1.3. Air contaminants formed

No additional information available

8.1.4. DNEL and PNEC

No additional information available

8.1.5. Control banding

No additional information available

8.2. Exposure controls

8.2.1. Appropriate engineering controls

No additional information available

8.2.2. Personal protection equipment

Personal protective equipment:

Avoid all unnecessary exposure.

Personal protective equipment symbol(s):

8.2.2.1. Eye and face protection

Eye protection:

Chemical goggles or safety glasses

8.2.2.2. Skin protection

Hand protection:

Wear protective gloves.

8.2.2.3. Respiratory protection

Respiratory protection:

Wear appropriate mask

8.2.2.4. Thermal hazards

No additional information available

8.2.3. Environmental exposure controls

Other information:

Do not eat, drink or smoke during use.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

| Physical state | : | Liquid |
| --- | --- | --- |
| Colour | : | Conforms to standard. |
| Odour | : | characteristic. |
| Odour threshold | : | Not available |
| Melting point | : | Not available |
| Freezing point | : | Not available |
| Boiling point | : | Not available |
| Flammability | : | Non flammable. |
| Lower explosion limit | : | Not available |
| Upper explosion limit | : | Not available |
| Flash point | : | > 93 °C |
| Auto-ignition temperature | : | Not available |
| Decomposition temperature | : | Not available |
| pH | : | Not available |
| Viscosity, kinematic | : | Not available |
| Solubility | : | Not available |
| Partition coefficient n-octanol/water (Log Kow) | : | Not available |
| Vapour pressure | : | Not available |
| Vapour pressure at 50°C | : | Not available |
| Density | : | Not available |
| Relative density | : | Not available |
| Relative vapour density at 20°C | : | Not available |
| Particle characteristics | : | Not applicable |

9.2. Other information

9.2.1. Information with regard to physical hazard classes

No additional information available

9.2.2. Other safety characteristics

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

No additional information available

10.2. Chemical stability

Not established.

10.3. Possibility of hazardous reactions

Not established.

10.4. Conditions to avoid

Direct sunlight. Extremely high or low temperatures.

10.5. Incompatible materials

Strong acids. Strong bases.

10.6. Hazardous decomposition products

fume. Carbon monoxide. Carbon dioxide.

SECTION 11: Toxicological information

11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

| Acute toxicity (oral) | : | Not classified |
| --- | --- | --- |
| Acute toxicity (dermal) | : | Not classified |
| Acute toxicity (inhalation) | : | Not classified |

| Verdox (88-41-5) |  |
| --- | --- |
| LD50 oral rat | 4600 mg/kg (Source: NLM_CIP) |
| LD50 oral | 4600 mg/kg |

| Geranyl acetate (105-87-3) |  |
| --- | --- |
| LD50 oral rat | 6330 mg/kg (Source: NLM_CIP) |

| Eugenol (97-53-0) |  |
| --- | --- |
| LD50 oral rat | 1930 mg/kg (Source: NZ_CCID) |
| LD50 oral | 2500 mg/kg bodyweight |
| LC50 Inhalation - Rat | > 2.58 mg/l/4h |

| Cinnamic aldehyde (104-55-2) |  |
| --- | --- |
| LD50 oral rat | 2220 mg/kg (Source: NLM_CIP) |
| LD50 oral | 2220 mg/kg |
| LD50 dermal rabbit | 1260 mg/kg (Source: EPA_HPV) |

| Neryl acetate (141-12-8) |  |
| --- | --- |
| LD50 oral rat | > 2000 mg/kg (Source: ECHA) |
| LD50 dermal rabbit | > 6 ml/kg (Source: ECHA_API) |

| (R)-p-mentha-1,8-diene; d-limonene (5989-27-5) |  |
| --- | --- |
| LD50 oral rat | 4400 mg/kg (Source: CHEMVIEW) |
| LD50 dermal rabbit | > 5 g/kg (Source: CHEMVIEW) |

| isobutyl acetate (110-19-0) |  |
| --- | --- |
| LD50 oral rat | 15400 mg/kg (Source: JAPAN_GHS) |
| LD50 dermal rabbit | > 17400 mg/kg (Source: NLM_CIP) |

| Hexyl cinnamic aldehyde (101-86-0) |  |
| --- | --- |
| LD50 oral rat | 3100 mg/kg (Source: NLM_CIP) |
| LD50 oral | 3100 mg/kg bodyweight |
| LD50 dermal rabbit | > 3000 mg/kg (Source: EPA_HPV) |
| LC50 Inhalation - Rat | > 5 mg/l/4h |

| Butylated hydroxytoluene (BHT) crystals (128-37-0) |  |
| --- | --- |
| LD50 oral rat | > 2930 mg/kg (Source: EPA_HPV) |
| LD50 dermal rat | > 2000 mg/kg (Source: JAPAN_GHS) |

| Triplal (Vertocitral) (68039-49-6) |  |
| --- | --- |
| LD50 oral | 2330 mg/kg |

| Toluene (108-88-3) |  |
| --- | --- |
| LD50 oral rat | 2600 mg/kg (Source: JAPAN_GHS) |
| LD50 dermal rabbit | 12000 mg/kg (Source: JAPAN_GHS) |
| LC50 Inhalation - Rat | 12.5 mg/l/4h |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| LD50 oral rat | 5.35 g/kg (Source: NLM_HSDB) |
| LD50 dermal rabbit | 9500 mg/kg (Source: NLM_CIP) |

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| LD50 oral rat | 2450 mg/kg (Source: NLM_CIP) |
| LD50 oral | 2830 mg/kg bodyweight |
| LD50 dermal rabbit | > 7940 mg/kg (Source: NLM_CIP) |
| LC50 Inhalation - Rat (Dust/Mist) | 1.5 mg/l/4h |

| Amyl formate (638-49-3) |  |
| --- | --- |
| LD50 oral rat | > 5 g/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 5000 mg/kg (Source: ECHA_API) |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| LD50 oral rat | 4720 mg/kg (Source: NZ_CCID) |
| LD50 dermal rat | > 5000 mg/kg (Source: ECHA_API) |
| LC50 Inhalation - Rat | > 71 mg/l (Exposure time: 1 h Source: ECHA_API) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| LD50 oral rat | 4890 mg/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 8100 mg/kg (Source: ECHA_API) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| LD50 oral rat | 3 g/kg (Source: NLM_HSDB) |
| LD50 oral | 4000 mg/kg bodyweight |
| LD50 dermal rabbit | 630 mg/kg (Source: NLM_HSDB) |

| butyric acid (107-92-6) |  |
| --- | --- |
| LD50 oral rat | 2 g/kg (Source: NLM_CIP) |
| LD50 oral | 1630 mg/kg bodyweight |
| LD50 dermal rabbit | 530 mg/kg (Source: NLM_HSDB) |

| Skin corrosion/irritation | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Serious eye damage/irritation | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Respiratory or skin sensitisation | : | May cause an allergic skin reaction. |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Germ cell mutagenicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Carcinogenicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Eugenol (97-53-0) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| (R)-p-mentha-1,8-diene; d-limonene (5989-27-5) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| Butylated hydroxytoluene (BHT) crystals (128-37-0) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| Toluene (108-88-3) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |

| Reproductive toxicity | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| STOT-single exposure | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| isobutyl acetate (110-19-0) |  |
| --- | --- |
| STOT-single exposure | May cause drowsiness or dizziness. |

| Toluene (108-88-3) |  |
| --- | --- |
| STOT-single exposure | May cause drowsiness or dizziness. |

| Amyl formate (638-49-3) |  |
| --- | --- |
| STOT-single exposure | May cause respiratory irritation. |

| STOT-repeated exposure | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| Toluene (108-88-3) |  |
| --- | --- |
| STOT-repeated exposure | May cause damage to organs through prolonged or repeated exposure. |

| Aspiration hazard | : | Not classified |
| --- | --- | --- |
| Additional information | : | Based on available data, the classification criteria are not met |

| (R)-p-mentha-1,8-diene; d-limonene (5989-27-5) |  |
| --- | --- |
| Hydrocarbon | Yes |

| Toluene (108-88-3) |  |
| --- | --- |
| Hydrocarbon | Yes |

11.2. Information on other hazards

11.2.1. Endocrine disrupting properties

No additional information available

11.2.2. Other information

| Potential adverse human health effects and symptoms | : | Based on available data, the classification criteria are not met |
| --- | --- | --- |

SECTION 12: Ecological information

12.1. Toxicity

| Hazardous to the aquatic environment, short–term (acute) | : | Not classified |
| --- | --- | --- |
| Hazardous to the aquatic environment, long–term (chronic) | : | Harmful to aquatic life with long lasting effects. |

| Eugenol (97-53-0) |  |
| --- | --- |
| LC50 - Fish [1] | 13 mg/l (Exposure time: 96 h - Species: Danio rerio [semi-static] Source: ECHA) |

| (R)-p-mentha-1,8-diene; d-limonene (5989-27-5) |  |
| --- | --- |
| LC50 - Fish [1] | 0.619 – 0.796 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 35 mg/l (Exposure time: 96 h - Species: Oncorhynchus mykiss Source: EPA) |

| isobutyl acetate (110-19-0) |  |
| --- | --- |
| LC50 - Fish [1] | 17 mg/l (Exposure time: 96 h - Species: Oryzias latipes Source: ECHA) |

| Butylated hydroxytoluene (BHT) crystals (128-37-0) |  |
| --- | --- |
| EC50 72h - Algae [1] | 6 mg/l (Species: Pseudokirchneriella subcapitata) |
| EC50 72h - Algae [2] | > 0.42 mg/l (Species: Desmodesmus subspicatus) |

| Toluene (108-88-3) |  |
| --- | --- |
| LC50 - Fish [1] | 15.22 – 19.05 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 12.6 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static] Source: EPA) |
| EC50 - Crustacea [1] | 5.46 – 9.83 mg/l (Exposure time: 48 h - Species: Daphnia magna [Static]) |
| EC50 - Crustacea [2] | 11.5 mg/l (Exposure time: 48 h - Species: Daphnia magna) |
| EC50 72h - Algae [1] | 12.5 mg/l (Species: Pseudokirchneriella subcapitata [static]) |
| EC50 96h - Algae [1] | > 433 mg/l (Species: Pseudokirchneriella subcapitata) |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| LC50 - Fish [1] | > 10000 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static]) |
| EC50 - Crustacea [1] | 1919 mg/l (Exposure time: 48 h - Species: Daphnia magna) |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| LC50 - Fish [1] | 2.2 – 2.5 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 4.12 – 6.2 mg/l (Exposure time: 96 h - Species: Lepomis macrochirus [static] Source: EPA) |
| EC50 - Crustacea [1] | 3 mg/l (Exposure time: 48 h - Species: Daphnia magna) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| LC50 - Fish [1] | 12 – 16.5 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| LC50 - Fish [1] | 306 – 334 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 88 mg/l (Exposure time: 96 h - Species: Pimephales promelas [static] Source: EPA) |

| butyric acid (107-92-6) |  |
| --- | --- |
| EC50 72h - Algae [1] | 46.7 mg/l (Species: Desmodesmus subspicatus) |

12.2. Persistence and degradability

| PEAR #EU7696F 25% in DPG |  |
| --- | --- |
| Persistence and degradability | Not established. |

| Verdox (88-41-5) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Geranyl acetate (105-87-3) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Eugenol (97-53-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Cinnamic aldehyde (104-55-2) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Neryl acetate (141-12-8) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| (R)-p-mentha-1,8-diene; d-limonene (5989-27-5) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| isobutyl acetate (110-19-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Hexyl cinnamic aldehyde (101-86-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Butylated hydroxytoluene (BHT) crystals (128-37-0) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Triplal (Vertocitral) (68039-49-6) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Toluene (108-88-3) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| isopentyl acetate (123-92-2) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Amyl formate (638-49-3) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

| butyric acid (107-92-6) |  |
| --- | --- |
| Persistence and degradability | Rapidly degradable |

12.3. Bioaccumulative potential

| PEAR #EU7696F 25% in DPG |  |
| --- | --- |
| Bioaccumulative potential | Not established. |

| Geranyl acetate (105-87-3) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.04 |

| Eugenol (97-53-0) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.83 (at 30 °C (at pH 5.5) |

| Cinnamic aldehyde (104-55-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.1065 (at 25 °C) |

| Neryl acetate (141-12-8) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 3.98 (at 37 °C (at pH 7.2) |

| (R)-p-mentha-1,8-diene; d-limonene (5989-27-5) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.38 (at 37 °C (at pH 7.2) |

| isobutyl acetate (110-19-0) |  |
| --- | --- |
| BCF - Fish [1] | (no significant bioconcentration) |
| Partition coefficient n-octanol/water (Log Pow) | 2.3 (at 25 °C (at pH 7) |

| Butylated hydroxytoluene (BHT) crystals (128-37-0) |  |
| --- | --- |
| BCF - Fish [1] | 230 – 2500 |
| Partition coefficient n-octanol/water (Log Pow) | 5.1 |

| Toluene (108-88-3) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.73 (at 20 °C (at pH 7) |

| Dipropylene glycol monomethyl ether (34590-94-8) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 0.35 (at 25 °C (at pH 7) |

| isopentyl acetate (123-92-2) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.7 (at 35 °C) |

| Diphenyl oxide (101-84-8) |  |
| --- | --- |
| BCF - Fish [1] | (470 dimensionless) |
| Partition coefficient n-octanol/water (Log Pow) | 4.21 (at 25 °C) |

| Amyl formate (638-49-3) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.786 (at 25 °C) |

| Alcohol C-10 (112-30-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 4.5 (at 25 °C (at pH 6) |

| Aldehyde C-6 (66-25-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 2.3 (at 25 °C (at pH 5) |

| Caproic acid (142-62-1) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.88 |

| butyric acid (107-92-6) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.1 (at 25 °C (at pH 3) |

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

No additional information available

12.6. Endocrine disrupting properties

No additional information available

12.7. Other adverse effects

| Additional information | : | Avoid release to the environment. |
| --- | --- | --- |

SECTION 13: Disposal considerations

13.1. Waste treatment methods

| Product/Packaging disposal recommendations | : | Dispose in a safe manner in accordance with local/national regulations. |
| --- | --- | --- |
| Ecological information | : | Avoid release to the environment. |
| HP Code | : | HP4 - “Irritant – skin irritation and eye damage:” waste which on application can cause skin irritation or damage to the eye. HP14 - “Ecotoxic:” waste which presents or may present immediate or delayed risks for one or more sectors of the environment |

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number or ID number |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.2. UN proper shipping name |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.3. Transport hazard class(es) |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.4. Packing group |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.5. Environmental hazards |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| No supplementary information available |  |  |  |  |

14.6. Special precautions for user

Overland transport

Not applicable

Transport by sea

Not applicable

Air transport

Not applicable

Inland waterway transport

Not applicable

Rail transport

Not applicable

14.7. Maritime transport in bulk according to IMO instruments

Not applicable

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

15.1.1. EU-Regulations

REACH Annex XVII (Restriction List)

| EU restriction list (REACH Annex XVII) |  |  |
| --- | --- | --- |
| Reference code | Applicable on | Entry title or description |
| 3(a) | (R)-p-mentha-1,8-diene; d-limonene ; isobutyl acetate ; Toluene ; isopentyl acetate ; Amyl formate ; Aldehyde C-6 | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 2.1 to 2.4, 2.6 and 2.7, 2.8 types A and B, 2.9, 2.10, 2.12, 2.13 categories 1 and 2, 2.14 categories 1 and 2, 2.15 types A to F |
| 3(b) | PEAR #EU7696F 25% in DPG ; Geranyl acetate ; Eugenol ; Cinnamic aldehyde ; Neryl acetate ; (R)-p-mentha-1,8-diene; d-limonene ; isobutyl acetate ; Hexyl cinnamic aldehyde ; Triplal (Vertocitral) ; Toluene ; Amyl formate ; Caproic acid ; butyric acid | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 3.1 to 3.6, 3.7 adverse effects on sexual function and fertility or on development, 3.8 effects other than narcotic effects, 3.9 and 3.10 |
| 3(c) | PEAR #EU7696F 25% in DPG ; Verdox ; Geranyl acetate ; Cinnamic aldehyde ; (R)-p-mentha-1,8-diene; d-limonene ; Hexyl cinnamic aldehyde ; Triplal (Vertocitral) ; Alcohol C-10 | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard class 4.1 |
| 40. | (R)-p-mentha-1,8-diene; d-limonene ; isobutyl acetate ; Toluene ; isopentyl acetate ; Amyl formate ; Aldehyde C-6 | Substances classified as flammable gases category 1 or 2, flammable liquids categories 1, 2 or 3, flammable solids category 1 or 2, substances and mixtures which, in contact with water, emit flammable gases, category 1, 2 or 3, pyrophoric liquids category 1 or pyrophoric solids category 1, regardless of whether they appear in Part 3 of Annex VI to Regulation (EC) No 1272/2008 or not. |
| 48. | Toluene | Toluene |

REACH Annex XIV (Authorisation List)

Contains no substance(s) listed on REACH Annex XIV (Authorisation List)

REACH Candidate List (SVHC)

Contains no substance(s) listed on the REACH Candidate List

PIC Regulation (Prior Informed Consent)

Contains no substance(s) listed on the PIC list (Regulation EU 649/2012 concerning the export and import of hazardous chemicals)

POP Regulation (Persistent Organic Pollutants)

Contains no substance(s) listed on the POP list (Regulation EU 2019/1021 on persistent organic pollutants)

Ozone Regulation (1005/2009)

Contains no substance(s) listed on the Ozone Depletion list (Regulation EU 1005/2009 on substances that deplete the ozone layer)

Council Regulation (EC) for the control of dual-use items

Contains no substance subject to the COUNCIL REGULATION (EC) for the control of dual-use items

Explosives Precursors Regulation (2019/1148)

Contains no substance(s) listed on the Explosives Precursors list (Regulation EU 2019/1148 on the marketing and use of explosives precursors)

Drug Precursors Regulation (273/2004)

Contains substance(s) listed on the Drug Precursors list (Regulation EC 273/2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances)

| Name | CN designation | CAS-No. | CN code | Category, Subcategory | Threshold | Annex |
| --- | --- | --- | --- | --- | --- | --- |
| Toluene |  | 108-88-3 | 2902 30 00 | Category 3 |  | Annex I |

15.1.2. National regulations

| France |  |  |
| --- | --- | --- |
| Occupational diseases. Code; Description. RG 4 BIS; Gastrointestinal disorders caused by benzene, toluene, xylenes and all products containing them. RG 84; Conditions caused by liquid organic solvents for professional use: saturated or unsaturated aliphatic or cyclic liquid hydrocarbons and mixtures thereof; liquid halogenated hydrocarbons; nitrated derivatives of aliphatic hydrocarbons; alcohols; glycols, glycol ethers; ketones; aldehydes; aliphatic and cyclic ethers, including tetrahydrofuran; esters; dimethylformamide and dimethylacetamine; acetonitrile and propionitrile; pyridine; dimethylsulfone and dimethylsulfoxide. |
| Germany |  |  |
| Water hazard class (WGK) | : | WGK 2, Significantly hazardous to water (Classification according to AwSV, Annex 1). |
| List of sensitizing substances (TRGS 907) | : | Contains sensitizing substances according TRGS 907. |
| Hazardous Incident Ordinance (12. BImSchV) | : | Is not subject to the Hazardous Incident Ordinance (12. BImSchV) |
| Netherlands |  |  |
| ABM category | : | A(3) - hazardous for aquatic organisms, may have longterm hazardous effects in aquatic environment |
| SZW-lijst van kankerverwekkende stoffen | : | Triplal (Vertocitral) is listed |
| SZW-lijst van mutagene stoffen | : | Triplal (Vertocitral) is listed |
| SZW-lijst van reprotoxische stoffen – Borstvoeding | : | None of the components are listed |
| SZW-lijst van reprotoxische stoffen – Vruchtbaarheid | : | None of the components are listed |
| SZW-lijst van reprotoxische stoffen – Ontwikkeling | : | Toluene is listed |
| Denmark |  |  |
| Classification remarks | : | Emergency management guidelines for the storage of flammable liquids must be followed |
| Danish National Regulations | : | Young people below the age of 18 years are not allowed to use the product Pregnant/breastfeeding women working with the product must not be in direct contact with the product |

15.2. Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

| Data sources | : | REGULATION (EC) No 1272/2008 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 December 2008 on classification, labelling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC, and amending Regulation (EC) No 1907/2006. |
| --- | --- | --- |
| Other information | : | None. |

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Acute Tox. 4 (Dermal) | Acute toxicity (dermal), Category 4 |
| Acute Tox. 4 (Oral) | Acute toxicity (oral), Category 4 |
| Aquatic Acute 1 | Hazardous to the aquatic environment – Acute Hazard, Category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment – Chronic Hazard, Category 1 |
| Aquatic Chronic 2 | Hazardous to the aquatic environment – Chronic Hazard, Category 2 |
| Aquatic Chronic 3 | Hazardous to the aquatic environment – Chronic Hazard, Category 3 |
| Asp. Tox. 1 | Aspiration hazard, Category 1 |
| Eye Dam. 1 | Serious eye damage/eye irritation, Category 1 |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 2 | Flammable liquids, Category 2 |
| Flam. Liq. 3 | Flammable liquids, Category 3 |
| H225 | Highly flammable liquid and vapour. |
| H226 | Flammable liquid and vapour. |
| H302 | Harmful if swallowed. |
| H304 | May be fatal if swallowed and enters airways. |
| H312 | Harmful in contact with skin. |
| H314 | Causes severe skin burns and eye damage. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H318 | Causes serious eye damage. |
| H319 | Causes serious eye irritation. |
| H335 | May cause respiratory irritation. |
| H336 | May cause drowsiness or dizziness. |
| H361d | Suspected of damaging the unborn child. |
| H373 | May cause damage to organs through prolonged or repeated exposure. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H411 | Toxic to aquatic life with long lasting effects. |
| H412 | Harmful to aquatic life with long lasting effects. |
| Repr. 2 | Reproductive toxicity, Category 2 |
| Skin Corr. 1B | Skin corrosion/irritation, Category 1, Sub-Category 1B |
| Skin Corr. 1C | Skin corrosion/irritation, Category 1, Sub-Category 1C |
| Skin Irrit. 2 | Skin corrosion/irritation, Category 2 |
| Skin Sens. 1 | Skin sensitisation, Category 1 |
| Skin Sens. 1A | Skin sensitisation, category 1A |
| Skin Sens. 1B | Skin sensitisation, category 1B |
| STOT RE 2 | Specific target organ toxicity – Repeated exposure, Category 2 |
| STOT SE 3 | Specific target organ toxicity – Single exposure, Category 3, Narcosis |

| The classification complies with | : | ATP 12 |
| --- | --- | --- |

Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.